SUPER COOLSCAN 9000 ED I/F PROTOCOL SPECIFICATIONS

1. FEATURES OF THE COMMUNICATION PROTOCOL IN THIS UNIT

The communication protocol of this unit conforms to the interface standard of IEEE1394 (SBP-2).

The specifications for each communication protocol are explained below.

1-1. SBP-2 Protocol Specifications

1-1-1. Outline

The host reads the configuration ROM of the target device, and ensures that the device has SBP-2 functions.

In SBP-2 protocol, the host only gives the pointer to the linked list of commands.  The target should fetch these commands, and execute them in proper order.  These SBP-2 transaction layers are supported in the link-layer-controller on this unit that conforms to IEEE1394-1995 and P1394a.

These include login, logout and reconnection of a management agent, the fetch engine that gets the linked list of ORB (Operation Request Block) from the logged-in nodes, flow control for data transfer, and the support for target node page table management.  Transaction layers analyze the ORB, and can extract CDB (Command Descriptor Block) and return it with status information to the logged-in node.

1-1-2. Supported functions

The functions that are supported in this unit are described below.

- Order command model

- Unsolicited status

- ABORT_TASK_SET management function

- TARGET_RESET management function

- NODE RESET

This unit does not support the following functions.

- Multiple host

- Unorder command model (command queue)

- ECA status

- SCSI command link function

- SET PASSWORD management function

- ABORT_TASK management function

- LOGICAL_UNIT_RESET management function

- Abort task in “rq_fmt=3” format

1-1-3. Session management, device detection, and data type

The outline of the sessions that are performed by the initiator for this unit is shown below.

- Log-in

- The address of the operation request block (ORB) with SCSI inquiry command (INQUIRY) is written to the ORB_POINTER register, and the SCSI inquiry command (INQUIRY) is issued.

- The command set implementation is assumed to be the SCSI Primary Commands (SPC).

- The ORB_POINTER writing is performed only after login, because the operating system uses all the doorbells.

- Log-out

1-1-3-1. Multiple logical unit

This unit is a single logical unit.

1-1-3-2. Password

The password is not used at the time of login.  Instead, when the initiator logs in, the password with 0 set to all figures is assumed.

The initiator does not set the password for the current implementation.  However, the command is supported for future use.

1-1-3-3. Exclusive login

The host retrieves the devices in order by using SBP-2 Device Type.

The target must abort the current task set in the very rare instance that the host log-out is terminated before the current task set.

1-1-3-4. Other commands

In the same manner, the host uses the following commands; ABORT_TASK_SET, TARGET_RESET, and RECONNECT.  These commands are used for the time-out request recovery between the sessions and reconnection after bus reset.

1-1-3-5. Device detection

The operating system uses only the configuration ROM to detect the device.  The target detection mechanism ‘depends on the command set’ according to ANSI SBP-2 specifications.  However, the host does not implement the mechanism.

1-1-4. Command execution

For the command execution model of this unit, ‘order model (the target executes one ORB at a time, and after each ORB is executed, the completion status is sent out)’ is used.

1-1-5. Task management

The host uses only ‘ABORT_TASK_SET’ when the task is aborted.  (The host does not use ‘Abort Task ORB’ or ‘rq_fmt=3’ format.)

When the target is reset, the host uses ‘TARGET_RESET’.  (The Logical Unit Reset command in the ANSI SBP-2 specifications is not used.)

1-1-6. Status responses

The SBP-2 module of the host always sets the notify bit to 1.  For the status format, refer to SBP-2 Annex B.

The host always sets the UNSOLICITED_STATUS_ENABLE register to ‘enable’.

If one or more status blocks are reserved, one status block must be sent out.

The UNSOLICITED_STATUS_ENABLE register is cleared.

The host enables UNSOLICITED_STATUS_ENABLE again.  The next UNSOLICITED STATUS BLOCK can be sent out at any time.

‘UNSOLICITED STATUS’ is the status that is transferred to the initiator by the target that is not related to a specific ORB.  (Data or management command)

1-1-7. CSR register

First, the host performs writing to the RESET_START register.  When it is not terminated, the host executes the low-level NODE RESET (IEEE1394-1995 Control Status Registers (CSR)).  The host does not use the STATE_CLEAR register.  In the same manner, the host does not read the AGENT_STATE register value.

If the request is out of time, the recovery starts the following steps.

- ABORT_TASK_SET is issued.

If ABORT_TASK_SET is succeeded, the operating system issues the request again and the process progresses.

- If ABORT_TASK_SET is failed, the operating system outputs TARGET_RESET.

If TARGET_RESET is succeeded, the request is sent again.

- If TARGET_RESET is failed, the operating system executes NODE RESET and after that, logs in again.

When the login is never terminated, the device may be broken.  And it is marked as the defective device.

Correspondence between SBP-2 and SCSI-2

| Action | SBP-2 | SCSI-2 |
| --- | --- | --- |
| Command abort | ABORT_TASK_SET management function | ABORT message |
| Hard reset | Writing to the RESET_START register TARGET_RESET management function NODE_RESET | BUS DEVICE RESET message |

1-1-8. Configuration ROM

This unit defines the configuration ROM as shown below based on the SBP-2 specifications.

Table 1-1-8-1 Configuration ROM format of this unit

| Bit Quadlet | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 1 0 | 1 1 | 1 2 | 1 3 | 1 4 | 1 5 | 1 6 | 1 7 | 1 8 | 1 9 | 2 0 | 2 1 | 2 2 | 2 3 | 2 4 | 2 5 | 2 6 | 2 7 | 2 8 | 2 9 | 3 0 | 3 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Length [04h] |  |  |  |  |  |  |  | CRC Length [11h] |  |  |  |  |  |  |  | ROM CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Bus Name=“1394” [31333934h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Node_Options [00FF5012h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Node_Vendor_ID [0090B5h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Chip_ID_Hi [40h] |  |  |  |  |  |  |  |
| 4 | Chip_ID_Lo [03h+Serial Number] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Directory Length [0005h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Root Directory CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Key [03h] |  |  |  |  |  |  |  | Module_Vendor_ID [0090B5h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Key [0Ch] |  |  |  |  |  |  |  | Node_Capabilities [0083C0h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Key [81h] |  |  |  |  |  |  |  | Offset to Leaf of Textual_Descriptor [00000Dh] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Key [04h] |  |  |  |  |  |  |  | Module_HW_Version [00500Ah] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Key [D1h] |  |  |  |  |  |  |  | Offset to Directory of Unit_Directory [000001h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Directory Length [0009h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Unit Directory CRC [44FEh] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Key [12h] |  |  |  |  |  |  |  | Unit_Spec_ID [00609Eh] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Key [13h] |  |  |  |  |  |  |  | Unit_SW_Version [010483h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Key [54h] |  |  |  |  |  |  |  | Management_Agent [00C000h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Key [3Ah] |  |  |  |  |  |  |  | Unit Characteristics [000A08h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Key [14h] |  |  |  |  |  |  |  | Logical_Unit_Number [060000h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Device_Type [06h] |  |  |  |  |  |  |  | LUN [0000h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Key [38h] |  |  |  |  |  |  |  | Command_Set_Spec_ID [00609h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Key [39h] |  |  |  |  |  |  |  | Command_Set [0104D8h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Key [17h] |  |  |  |  |  |  |  | Model_ID [004002h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Key [81h] |  |  |  |  |  |  |  | Offset to Leaf of Textual_Descriptor [000007h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | Leaf Length [0005h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Text Leaf CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | Spec_Type [00h] |  |  |  |  |  |  |  | Specifier_ID [000000h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | Language_ID [00000000h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 to 26 | (“ Nikon ”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 | Leaf Length [0006h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Text Leaf CRC [0FDDh] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | Spec_Type [00h] |  |  |  |  |  |  |  | Specifier_ID [000000h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 | Language_ID [00000000h] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 to 33 | (“ LS-9000 ED ”) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remarks: Directory structure of the configuration ROM

Bus information block			(Quadlet 0 to 4)

|----- Root directory			(Quadlet 5 to 10)

|- Unit directory			(Quadlet 11 to 20)

|----- Text descriptor leaf		(Quadlet 21 to 26)

|- Text descriptor leaf		(Quadlet 27 to 33)

1-1-9. Commands of this unit

The commands that are executed by this unit are shown in table 1-1-9-1.

Table 1-1-9-1 List of the commands of this unit

| Command name | Operation code | Type | Service (phase) transition |
| --- | --- | --- | --- |
| TEST UNIT READY | 00h | M | C - S |
| INQUIRY | 12h | M | C - Din - S |
| MODE SELECT (6) | 15h | O | C - Dout - S |
| RESERVE UNIT | 16h | M | C - S |
| RELEASE UNIT | 17h | M | C - S |
| MODE SENSE (6) | 1Ah | O | C - Din - S |
| SCAN | 1Bh | M | C - S |
| RECEIVER DIAGNOSTIC RESULTS | 1Ch | M | C - S |
| SEND DIAGNOSTIC | 1Dh | M | C - S |
| SET WINDOW | 24h | M | C - Dout - S |
| GET WINDOW | 25h | O | C - Din - S |
| READ | 28h | M | C - Din - S |
| SEND | 2Ah | O | C - Dout - S |
| ABORT | C0h | V | C - S |
| EXECUTE | C1h | V | C - S |
| SET PARAMETER | E0h | V | C - Dout - S |
| GET PARAMETER | E1h | V | C - Din - S |

Remarks

M	: Mandatory command in the SCSI-2 standard

O	: Option command in the SCSI-2 standard

V	: Command that is originally specified in this unit

Service (phase) explanation

C	: Command service (phase)

Din	: Data in service (phase)

Dout	: Data out service (phase)

S	: Status service (phase)

Note) The presence of data service (phase) described in the example is in the case that the transfer length contains non-zero value.

1-1-10. Status of this unit

The statuses that are transferred by this unit are shown below.

The status is transferred when each command is terminated if the command is not terminated by any of the conditions below.

- In the case of power reset

- In the case of hard reset

- When writing to the RESET_START register is performed

- TARGET_RESET management function

- NODE RESET (IEEE1394-1995 Control Status Registers (CSR))

- When the ABORT_TASK_SET management function is received

- In the case of log-out

Table 1-1-10-1 Status byte code of this unit

| Bit of the status byte |  |  |  |  |  |  |  |  | Status |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |  |
| R |  | R | 0 | 0 | 0 | 0 | 0 | R | GOOD | [00h] |  |
| R |  | R | 0 | 0 | 0 | 0 | 1 | R | CHECK CONDITION | [02h] |  |
| R |  | R | 0 | 0 | 1 | 0 | 0 | R | BUSY | [08h] |  |
| R |  | R | 0 | 1 | 1 | 0 | 0 | R | RESERVATION CONFLICT | [18h] |  |
| Key: R - Reserved bit (set to 0) |  |  |  |  |  |  |  |  |  |  |

When the status is ‘GOOD’, only the first 2 quadlets of the status ORB are transferred.

When the status is other than ‘GOOD’ (CHECK CONDITION and so on), the eight quadlets with sense data are transferred as shown below.

Table 1-1-10-2 Status of this unit

| Bit Quadlet | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 1 0 | 1 1 | 1 2 | 1 3 | 1 4 | 1 5 | 1 6 | 1 7 | 1 8 | 1 9 | 2 0 | 2 1 | 2 2 | 2 3 | 2 4 | 2 5 | 2 6 | 2 7 | 2 8 | 2 9 | 3 0 | 3 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Src |  | Re sp |  | D | Len |  |  | Sbp_status |  |  |  |  |  |  |  | ORB_offset_hi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | ORB_offset_lo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | r |  |
| 2 | sfmt |  | Status |  |  |  |  |  | V | M | E | I | Sense_key |  |  |  | Sense_code |  |  |  |  |  |  |  | Sense_qualifier |  |  |  |  |  |  |  |
| 3 | Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | CDB-dependent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Fru |  |  |  |  |  |  |  | Sense_key-dependent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6, 7 | Vendor-dependent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Quadlet 2

- Status (bit 2 to 7)

Indicates the status code in table 1-1-10-1.

- Sense_key (bit 12 to 15)

Indicates the sense key.

- Sense_code (bit 16 to 23)

Indicates the additional sense code (ASC).

- Sense_qualifier (bit 24 to 31)

Indicates the additional sense code qualifier (ASCQ).

Quadlet 5

- Fru (bit 0 to 7)

Indicates the detailed error information.

1-1-11. Command processing and exception status

1-1-11-1. CA status, auto sense, REQUEST SENSE command

For the sense data clearance, when the status block is returned by the auto sense function of the SBP-2 target implement, the CA status condition is automatically cleared.

The SBP-2 target can transfer the status, together with the sense data.

When the CA status occurs in the target (CHECK CONDITION status is returned), the status with the sense data is transferred.

The initiator that detected the CA status (received the CHECK CONDITION status) does not have to execute the REQUEST SENSE command.

1-1-11-2. I/O process in the queue

Not supported by this unit.

1-1-11-3. Unit attention status

This unit generates the unit attention status for each initiator in the following cases.

- After the power is turned ON (UA without film is not transferred)

- When the attached holder is removed (UA without film is not transferred)

- When the holder is exchanged

1-1-11-4. Log-out processing

When the log-out occurs, this unit moves the fetch agent of the target to RESET and operates as follows.

- The command is aborted and the status when the log-out is performed succeeds.

- Perform the same processing as that in the case of SCSI-ABORT command.

- Perform the same operations as TARGET_RESET (hard reset of the target by the host).

# 2. COMMAND EXPLANATIONS

Each command is explained below.

In the explanations, the common error responses are as shown in the table below.

| Common error | Sense data | Remarks |
| --- | --- | --- |
| 1 | INVALID FIELD IN CDB (Some illegal data exists in the CDB.) 05h-24h-00h-00h | Terminates with CHECK CONDITION status. |
| 2 | INVALID FIELD IN PARAMETER LIST (Some illegal data exists in the parameter.) 05h-26h-00h-00h | Terminates with CHECK CONDITION status. |

# Other error responses are explained individually in the explanations below.

The values in [ ] in the table show the permissible values or recommended values of this unit in the command description block and in the parameter, or the values that are returned by this unit in the response data.

2-1. TEST UNIT READY Command

# Table 2-1-1 TEST UNIT READY command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [00h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 to 4 | Reserved [0] |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

The TEST UNIT READY command provides a means to check if the logical unit is ready.

Table 2-1-2 shows the responses corresponding to the TEST UNIT READY command.  A response that has higher priority (RESERVATION CONFLICT, for example) may be made.

# Table 2-1-2 Preferred Test Unit Ready Responses

| Status | Sense code |
| --- | --- |
| GOOD | No Additional Sense Information 00h-00h-00h-00h (No error) |
| Check Condition | Logical Unit Not Supported 05h-25h-00h-00h (An LUN other than 0 was specified.) |
| Check Condition | Logical Unit Is In Process Of Becoming Ready 02h-04h-01h-00h (During the execution of the operation activation command) 02h-04h-01h-01h (During loading/ejection of the object to be scanned) 02h-04h-01h-02h (During the measurement of the correction data) |
| Check Condition | Logical Unit Not Ready, Cause Not Reportable 02h-04h-02h-00h (The internal mechanical error occurred.) |
| Check Condition | Logical Unit Not Ready, Manual Intervention Required 02h-04h-03h-06h (FH-869GR: The mask is not set.) 02h-04h-03h-07h (Undefined holder) |
| Check Condition | Logical Unit Does Not Respond To Selection 02h-05h-00h-00h (The operation is possible, but the initialization operation in the unit is not completed because the power is just turned ON.) |
| Check Condition | Medium Not Present 02h-3Ah-00h-01h (The holder is not inserted.) |
| Check Condition | 06h-xxh-xxh-xxh Unit Attention |
| Check Condition | Data Phase Error 0Bh-4Bh-00h-00h (Unexpected error during Data Phase) |
| Check Condition | Overlapped Commands Attempted 0Bh-4Eh-00h-00h (The unit is selected by the same initiator while disconnecting.) |

2-2. INQUIRY Command

# Table 2-2-1-1 INQUIRY command

| Bit Byte | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [12h] |  |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  |  | Reserved [0] |  |  |  | EVPD [0, 1] |
| 2 | Page code [0] |  |  |  |  |  |  |  |  |
| 3 | Reserved [0] |  |  |  |  |  |  |  |  |
| 4 | Allocation length [Recommended value 36d] |  |  |  |  |  |  |  |  |
| 5 | Reserved [0] |  |  | Control byte [0] |  |  |  |  |  |

- The INQUIRY command sends information regarding parameters of this unit and its attached logical units to the initiator.

- An EVPD (Enable Vital Product Data) bit of zero specifies that this unit shall send the standard INQUIRY data to the initiator.  An EVPD bit of one indicates that this unit shall send the VPD (Vital Product Data) specified by the page code field to the initiator.

- The page code field specifies the kind of VPD information when EVPD bit has a value of one.  An EVPD bit of zero specifies that the page code is invalid.  Table 2-2-1-2 shows the list of page codes supported in this unit.

- The INQUIRY command shall return with CHECK CONDITION status only when this unit cannot return the requested INQUIRY data.

- If an INQUIRY command is received from an initiator with a pending unit attention condition (i.e., before this unit reports CHECK CONDITION status), this unit shall perform the INQUIRY command and shall not clear the unit attention condition.

Table 2-2-1-2 Page code field list

| VPD | Page code |  |  | Sub-section | Attached holder |
| --- | --- | --- | --- | --- | --- |
| 0 | Standard INQUIRY data |  | 00h (*1) | 2-2-1 |  |
| 1 | VPD informa-tion | Page code list | 00h | 2-2-2-1 |  |
|  |  | FRU ASCII information | 01h (unused) | 2-2-2-2 | - |
|  |  |  | 10h |  | Undefined holder |
|  |  |  | 11h (unused) |  | - |
|  |  |  | 12h |  | FH-816 |
|  |  |  | 13h |  | FH-8G1 |
|  |  |  | 14h |  | FH-835M |
|  |  |  | 15h |  | FH-835S |
|  |  |  | 16h |  | FH-869M |
|  |  |  | 17h |  | FH-869S |
|  |  |  | 18h |  | FH-869G |
|  |  |  | 19h |  | FH-869GR 6*4.5 |
|  |  |  | 1Ah |  | FH-869GR 6*6 |
|  |  |  | 1Bh |  | FH-869GR 6*7 |
|  |  |  | 1Ch |  | FH-869GR 6*8 |
|  |  |  | 1Dh |  | FH-869GR 6*9 |
|  |  |  | 1Eh |  | FH-869GR panorama 58 |
|  |  |  | 1Fh |  | FH-869GR panorama 65 |
|  |  |  | 40h (unused) |  | - |
|  |  |  | 41h (unused) |  | - |
|  |  |  | 42h (unused) |  | - |
|  |  |  | 50h (unused) |  | - |
|  |  |  | 60h (unused) |  | - |
|  |  |  | 61h (unused) |  | - |
|  |  | Address information | C1h | 2-2-2-3 |  |
|  |  | Additional address information | C8h | 2-2-2-6 |  |
|  |  |  | C9h |  | FH-816 |
|  |  |  | CAh |  | FH-816 |
|  |  |  | CBh |  | FH-816 |
|  |  | SET WINDOW function | D1h | 2-2-2-4 |  |
|  |  | Other information | E1h | 2-2-2-5 |  |
|  |  | Unused page | F0h | - | - |
|  |  |  | F8h | - | - |

*1 Page code field value of the INQUIRY command that is transferred from the initiator

2-2-1. Standard INQUIRY data of this unit

# The standard INQUIRY data of this unit is the mandatory data of 36-byte length only.

# Table 2-2-2 Standard INQUIRY data format of this unit

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | RMB [1] | Device-Type Modifier [0] |  |  |  |  |  |  |
| 2 | ISO Version [0] |  | ECMA Version [0] |  |  | ANSI-Approved Version [2=010b] |  |  |
| 3 | AENC [0] | TrmIOP [0] | Reserved [0] |  | Response Data Format [2=0010b] |  |  |  |
| 4 | Additional Length (n-4) [1Fh=31d] |  |  |  |  |  |  |  |
| 5, 6 | Reserved [0] |  |  |  |  |  |  |  |
| 7 | RelAdr [0] | WBus32 [0] | WBus16 [0] | Sync [0] | Linked [0] | Reserved [0] | CmdQue [0] | SftRe [0] |
| 8 to 15 | Vendor Identification [Nikon] |  |  |  |  |  |  |  |
| 16 to 31 | Product Identification [LS-9000 ED ] |  |  |  |  |  |  |  |
| 32 to 35 | Product Revision Level Example: [1.00] |  |  |  |  |  |  |  |

*1 When an invalid logical unit selection is performed

The RMB (Removable medium) bit is set to one.

The Vendor Identification field contains eight bytes of ASCII data identifying the vendor of the product.  In this field, the data shall be left aligned and unused bytes shall be filled with space characters (20h).

The Product Identification field contains sixteen bytes of ASCII data defined by the vendor.  In this field, the data shall be left aligned and unused bytes shall be filled with space characters (20h).

The Product Revision Level field contains four bytes of ASCII data defined by the vendor.  In this field, the data shall be left aligned and unused bytes shall be filled with space characters (20h).

2-2-2. VPD information

When the EVPD (Enable Vital Product Data) bit is set to one, this unit sends VPD (Vital Product Data) specified in the page code field to the initiator.

Byte 0

This byte has the same Peripheral Qualifier and Peripheral Device Type as the standard Inquiry data.

# Byte 1

It contains a page code.  Cxh, Dxh, Exh, and Fxh of the page code indicate the adapter information, Set Window information, other information, and the experimental information, respectively.

# Byte 3

It contains the length of the page data from byte 4 onwards.

Even if all the page data cannot be transferred because the CDB data length is short, this byte indicates the whole byte length of the defined page data.

# Byte 4 and after

These bytes are defined on each page.

If the VPD information is requested when the adapter is not installed, this unit transfers the information with Peripheral Qualifier set to one, Peripheral Device Type set to 6, page code that is requested, and the page length set to one.  The VPD information of byte 5 is the invalid data.

2-2-2-1. Page code list page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [00h] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [m-3] |  |  |  |  |  |  |  |
| 4 to m | Page code list [m-4] |  |  |  |  |  |  |  |

*1  When an invalid logical unit selection is performed

This shows the list of information page codes that are supported by this unit.

Byte 4  Page code list

In this field, the information page codes that are supported by this unit are shown in units of one byte length in order starting from page code 00h.

This unit supports 00h, 01h, 40h, 41h, 42h, 50h, 60h, 61h, C1h, D1h, E1h, F0h, F8h, IDs of the attached holder (10h to 1Fh), and the additional address page.

For the additional address page, C8h, C9h, CAh, and CBh are supported when the attached holder is FH-816; and C8h is supported in other cases.

Note) On the above supported pages, 01h, 40h, 41h, 42h, 50h, 60h, 61h, F0h, and F8h are not used.

2-2-2-2. FRU ASCII information page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [01 to 7Fh] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [m-3] |  |  |  |  |  |  |  |
| 4 | ASCII data length [m-4] |  |  |  |  |  |  |  |
| 5 to m | ASCII information |  |  |  |  |  |  |  |

*1 When an invalid logical unit selection is performed

This page contains the unit information such as the adapter name, the unit name of a stage or a motor, and the name of the experimental parameter as the ASCII character strings.

The page code becomes the ID of the unit information that is shown in the ASCII information.

# Byte 4  ASCII data length

This field specifies the byte length of the ASCII information.

# Byte 5 and after  ASCII information

This field specifies the unit information as the ASCII character strings.

Only the graphic codes (20h to 7Eh) are used in the ASCII character strings, and the last one character in each line is set to the NULL code (00h).

This unit supports the unit information shown in table 2-2-2-2-1.

# Table 2-2-2-2-1 Holder ID and holder name

| Page code (ID) | Attached holder | ASCII information | Descriptions |
| --- | --- | --- | --- |
| 10h | Undefined holder | Unknown | Undefined holder |
| 12h | FH-816 | 16mm Film | 16-mm film holder |
| 13h | FH-8G1 | Praparat | Praparat holder |
| 14h | FH-835M | 35mm Mount Film | 35-mm mount film holder |
| 15h | FH-835S | 35mm Strip Film | 35-mm strip film holder |
| 16h | FH-869M | Brownie Mount Film | Brownie mount film holder |
| 17h | FH-869S | Brownie Strip Film | Brownie strip film holder |
| 18h | FH-869G | Brownie Strip Film with G | Brownie strip film holder with glass |
| 19h | FH-869GR 6*4.5 | 6*4.5 Film | Rotation holder 6*4.5 |
| 1Ah | FH-869GR 6*6 | 6*6 Film | Rotation holder 6*6 |
| 1Bh | FH-869GR 6*7 | 6*7 Film | Rotation holder 6*7 |
| 1Ch | FH-869GR 6*8 | 6*8 Film | Rotation holder 6*8 |
| 1Dh | FH-869GR 6*9 | 6*9 Film | Rotation holder 6*9 |
| 1Eh | FH-869GR panorama 58 | Panorama 58 | Rotation holder panorama 58 |
| 1Fh | FH-869GR panorama 65 | Panorama 65 | Rotation holder panorama 65 |

2-2-2-3. Address information page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [C1h] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [87d=57h] |  |  |  |  |  |  |  |
| 4 | SCSI function support (SCSI data transfer function) [1] |  |  |  |  |  |  |  |
| 5, 6 | Window descriptor block length [58=003Ah] |  |  |  |  |  |  |  |
| 7, 8 | Set parameter descriptor block length (Length of the SET PARAMETER command parameter in bytes) [15=000Fh] |  |  |  |  |  |  |  |
| 9, 10 | General SCSI Buffer Size (SCSI data transfer buffer size. Unit: byte) [0] |  |  |  |  |  |  |  |
| 11, 12 | Image Buffer Size (Unit: KB) [256=0100h] |  |  |  |  |  |  |  |
| 13 | Number of equipped Unit (the number of units that can be attached simultaneously) [1] |  |  |  |  |  |  |  |
| 14 | Unit Name ID [01h] |  |  |  |  |  |  |  |
| 15 | Current Holder Name ID (ID number of the attached holder) |  |  |  |  |  |  |  |
| 16 | Coordinate base information (resolution type and scanning that are supported) [42h] |  |  |  |  |  |  |  |
| 17 | Addressing Kind (addressing type that is supported) [12h] |  |  |  |  |  |  |  |
| 18, 19 | X-Optical Resolution (Unit: dpi) [4000=0FA0h] |  |  |  |  |  |  |  |
| 20, 21 | X-Maximum Resolution (Unit: dpi) [4000=0FA0h] |  |  |  |  |  |  |  |
| 22, 23 | X-Minimum Resolution (Unit: dpi) [666=029Ah] |  |  |  |  |  |  |  |
| 24 to 27 | X-Maximum Set Window Address (Window descriptor X-axis offset address maximum value) [9999=270Fh] |  |  |  |  |  |  |  |
| 28 to 31 | X-Minimum Set Window Address (Window descriptor X-axis offset address minimum value) [0] |  |  |  |  |  |  |  |
| 32 to 35 | X-Offset for first image’s address (X-axis scanning start position offset address) [0] |  |  |  |  |  |  |  |
| 36 to 39 | X-Set Window boundary (Maximum window width value of the X-axis window descriptor) [10000=2710h] |  |  |  |  |  |  |  |
| 40, 41 | Y-Optical Resolution (Unit: dpi) [4000=0FA0h] |  |  |  |  |  |  |  |

| 42, 43 | Y-Maximum Resolution (Unit: dpi) [4000=0FA0h] |
| --- | --- |
| 44, 45 | Y-Minimum Resolution (Unit: dpi) [333=014Dh] |
| 46 to 49 | Y-Maximum Set Window Address (Window descriptor Y-axis offset address maximum value) |
| 50 to 53 | Y-Minimum Set Window Address (Window descriptor Y-axis offset address minimum value) [0] |
| 54 to 57 | Y-Offset for first image’s address (Y-axis scanning start position offset address) [0] |
| 58 to 61 | Y-Set Window boundary (Maximum window width value of the Y-axis window descriptor) |
| 62 to 65 | Y-Another world maximum Address (Maximum address in the sub-scanning direction outside the specified address) [0] |
| 66 to 69 | Y-Another world minimum Address (Minimum address in the sub-scanning direction outside the specified address) [0] |
| 70, 71 | Maximum Thumbnail Resolution (maximum resolution in thumbnail scanning. Unit: dpi) [83=0053h] |
| 72, 73 | Minimum Thumbnail Resolution (minimum resolution in thumbnail scanning. Unit: dpi) [83=0053h] |
| 74 | Maximum Image count (maximum number of frames that can be scanned) |
| 75 | Actual including image count (the number of medium frames that are currently set) |
| 76, 77 | Minimum Focusing Address (minimum address of the focus position) [0] |
| 78, 79 | Maximum Focusing Address (maximum address of the focus position) [450=01C2h] |
| 80, 81 | Lamp warm-up maximum time (maximum time for lamp warming-up) [0] |
| 82 | A/D bit depth (depth of bits for an A/D converter) [16=10h] |
| 83, 84 | CCD Pixel Number (the number of effective pixels in the CCD. For the CCD in which the number of effective pixels differs in each color, the maximum value is set.) [10000=2710h] |
| 85 | Line Gap Count (the number of pixels between lines of the CCD) [12=0Ch] |
| 86 | CCD Line Number (the number of lines in the CCD) [03h] |
| 87, 88 | Reserved |
| 89, 90 | Reserved |

*1 When an invalid logical unit selection is performed

Byte 4  SCSI function support

This field specifies the SCSI data transfer function.

Setting each bit to zero indicates that the function is not supported by this unit, and setting each bit to one indicates that the function is supported by this unit.  In this unit, this field is set to 01h.

| Bit | Descriptions | Support of this unit |
| --- | --- | --- |
| 0 | Unused | 1 |
| 1 | Image reading (READ command) must be performed in units of [Data of one line in bytes * number of colors]. | 0 |
| 2 | Image reading (READ command) must be performed in units of [Data of one line in bytes]. | 0 |
| 3 to 6 | Reserved | 0 |
| 7 | Extend bit | 0 |

Byte 5, 6  Window descriptor block length

This field specifies the length of the window descriptor in bytes.  In this unit, this field is set to 58d.

Byte 7, 8  Set parameter descriptor block length

This field specifies the length of the SET PARAMETER command parameter in bytes.  In this unit, this field is set to 15d.

Byte 9, 10  General SCSI Buffer Size

This field specifies the data size that is used for the SCSI data transfer in bytes.  Zero indicates that the buffer size is not limited.  In this unit, this field is set to zero.

Byte 11, 12  Image Buffer Size

This field specifies the image buffer size in kilobytes.  In this unit, this field is set to 256d.

Byte 13  Number of equipped Unit

This field specifies the number of units that can be attached to this unit simultaneously.  In this unit, this field is set to one.

Byte 14  Unit Name ID

This field specifies the ID number of the adapter that is currently attached.  In this unit, this field is set to 1.

Byte 15  Current Holder Name ID

This field specifies the ID number of the attached holder.

Byte 16  Coordinate base information

Each bit in this field specifies the resolution type and the reading method that are supported.

| Bit0, 1 | Resolution type [2] | Setting this bit to 0 indicates that reading can be performed in continuous resolution. Setting this bit to 1 indicates that reading can be performed only in the resolution of each pitch. Setting this bit to 2 indicates that reading can be performed in the Line Gap Count measure pitch. For the pitch, refer to the explanation of the GET WINDOW command. |
| --- | --- | --- |
| Bit2 | X Origin Reversed [0] | Setting this bit to 1 indicates that the main-scanning direction origin is reversed (at the right end of the medium). |
| Bit3 | Y Origin Reversed [0] | Setting this bit to 1 indicates that the sub-scanning direction origin is reversed (at the bottom end of the medium). |
| Bit4 | Thumbnail Order Reversed [0] | Setting this bit to 0 indicates that the thumbnail image is stored in the normal direction (first frame->last frame). Setting this bit to 1 indicates that the thumbnail image is stored in the reversed direction (last frame->first frame). |
| Bit5 | Reserved [0] | This bit is set to 0 in this unit. |
| Bit6 | Additional Coordinate Information [1] | This bit is set to 1 in this unit. |
| Bit7 | Extend bit [0] | This bit is set to 0 in this unit. |

# Byte 17  Addressing Kind

This field specifies the addressing type that is supported.  The addressing of the bit to which 1 is set is supported.

| Bit | Descriptions |  |  | Support of this unit |  |
| --- | --- | --- | --- | --- | --- |
| 0 | The Set Window address is the same as the medium position address. |  |  | 0 |  |
| 1 | The Set Window address is the same as the address of the mechanical block. |  |  | 1 |  |
| 2 | Specifying the scanning range over two or more frames is prohibited. |  |  | 0 |  |
| 3 | Reserved |  |  | 0 |  |
| 4 | The position of the medium can be operated. |  |  | 1 |  |
| 5 | The mechanical block position can be operated. |  |  | 0 |  |
| 6 | Reserved |  |  | 0 |  |
| 7 | Extension bit |  |  | 0 |  |

# Byte 24 to 27  X-Maximum Set Window Address

# This field specifies the maximum value of the X-axis offset address for the window descriptor.

# If this address is the same as X-Minimum Set Window Address in byte 28 to 31, it means that this unit cannot specify the optional reading range in the X-axis direction.

# The initiator must set the reading range in the X-axis direction to the value of X-Set Window boundary (Maximum window width value of the X-axis window descriptor) in byte 36 to 39 when reading is performed.

# Byte 46 to 49  Y-Maximum Set Window Address

# This field specifies the maximum value of the Y-axis offset address for the window descriptor.

# If this address is the same as Y-Minimum Set Window Address in byte 50 to 53, it means that this unit cannot specify the optional reading range in the Y-axis direction.

# The initiator must set the reading range in the Y-axis direction to the value of Y-Set Window boundary (Maximum window width value of the Y-axis window descriptor) in byte 58 to 61 when reading is performed.

# Byte 75  Actual including image count

This field specifies the number of frames of the medium that is currently set.

# Byte 86  CCD Line Number

This field specifies the number of lines in the CCD.  When 0 is set or no value is sent to this field, ‘3 lines’ is set.  In this unit, this field is set to 3.

2-2-2-4. SET WINDOW function page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [D1h] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [24d=18h] |  |  |  |  |  |  |  |
| 4 | Scanning Kind Support [03h] |  |  |  |  |  |  |  |
| 5 | Scan Mode Support [16h] |  |  |  |  |  |  |  |
| 6 | Color Interleaving Support (color order for data transfer) [42h] |  |  |  |  |  |  |  |
| 7 | Color Component [06h] |  |  |  |  |  |  |  |
| 8 | Color Ordering1 [0] |  |  |  |  |  |  |  |
| 9 | Color Ordering2 [0] |  |  |  |  |  |  |  |
| 10 | Output Bit Depth/Dot a Color Support (the number of bits in one-color data) [20h] |  |  |  |  |  |  |  |
| 11 | Number of Setup Mode [0] |  |  |  |  |  |  |  |
| 12 | Digital Image Control Support [0] |  |  |  |  |  |  |  |
| 13 | Additional length for Digital Control Information [0] |  |  |  |  |  |  |  |
| 14 | Analog Control Support [40h] |  |  |  |  |  |  |  |
| 15 | Additional length for Analog Control Information [9] |  |  |  |  |  |  |  |
| 16 to 24 | for the First Supported Control (exposure value control parameter) Byte 16 Bytes a Value for the control (parameter length in bytes) [4] Byte 17 to 20 Minimum Value for the First Control [00000001h] Byte 21 to 24 Maximum Value for the First Control [03FFFFFFh] |  |  |  |  |  |  |  |
| 25 | Filter Support [0] |  |  |  |  |  |  |  |
| 26 | Matrix Support [0] |  |  |  |  |  |  |  |
| 27 | Halftone Support [0] |  |  |  |  |  |  |  |

*1 When an invalid logical unit selection is performed

# Byte 4  Scanning Kind Support

This field specifies the image scanning types that are supported.

This unit supports Image Scanning and Thumbnail Scanning.

| Bit | Type | Explanations of operation | Support of this unit |
| --- | --- | --- | --- |
| 0 | Image Scanning | Normal image scanning | 1 |
| 1 | Thumbnail Scanning | Thumbnail image scanning | 1 |
| 2 | Set up Scanning | Prescan Scanning for deciding the optimal integral time and gain, etc. | 0 |
| 3 | Reserved | Reserved | - |
| 4 | Reserved | Reserved | - |
| 5 | Auto Exposure Scanning | Scanning for deciding the integral time at which the output value becomes the AE Value that is set in each color | 0 |
| 6 | AE with WB Scanning | Scanning for deciding the integral time at which the maximum value of the output values in each color becomes the AE Value that is set with the white balance maintained | 0 |
| 7 | Extend bit | Extension bit [0] | 0 |

Byte 5  Scan Mode Support

This field specifies the scanning mode.

Normal Quality Scan, High Speed Scan, Multiple Reading Scan, and Reverse direction Scanning Supported are supported.

| Bit0 | High Quality Scan | [0] |
| --- | --- | --- |
| Bit1 | Normal Quality Scan | [1] |
| Bit2 | High Speed Scan | [1] |
| Bit3 | Reserved | [0] |
| Bit4 | Multiple Reading Scan | [1] |
| Bit5 | Reserved | [0] |
| Bit6 | Reverse direction Scanning Supported | [0] |
| Bit7 | Extend bit | [0] |

# Byte 6  Color Interleaving Support

This field specifies the color order for data transfer.

‘Line without CCD distance’ and ‘Multi line Simultaneous reading’ are supported.

| Bit0 | Pixel without CCD distance | [0] |
| --- | --- | --- |
| Bit1 | Line without CCD distance | [1] |
| Bit2 | Plane | [0] |
| Bit3 | Reserved | [0] |
| Bit4 | Pixel with CCD distance | [0] |
| Bit5 | Line with CCD distance | [0] |
| Bit6 | Multi line Simultaneous reading | [1] |
| Bit7 | Extend bit | [0] |

Byte 7  Color Component

This field specifies the color composition to be scanned.  Dropout Color and R-G-B are supported.

| Bit0 | Neutral Gray Scale | [0] |
| --- | --- | --- |
| Bit1 | Dropout Color | [1] |
| Bit2 | R-G-B | [1] |
| Bit3 | C-M-Y | [0] |
| Bit4 | Reserved | [0] |
| Bit5 | Reserved | [0] |
| Bit6 | Reserved | [0] |
| Bit7 | Extend bit | [0] |

# Byte 8  Color Ordering1

This field specifies the color ordering in which this unit can read the data.

In the case of R-G-B scanning, the setting is R=1, G=2, B=3.  In the case of C-M-Y, the setting is C=1, M=2, Y=3.

Bit 0 to 3 specify the color that can be scanned as the first color.  0 indicates that all colors can be scanned as the first color.

Bit 4 to 7 specify the color that can be scanned as the second color.  0 indicates that all colors can be scanned as the second color.

| Bit0-3 | First component color |
| --- | --- |
| Bit4-7 | Second component color |

# Byte 9  Color Ordering2

This field specifies the color ordering in which this unit can read the data.

In the case of R-G-B scanning, the setting is R=1, G=2, B=3.  In the case of C-M-Y, the setting is C=1, M=2, Y=3.

Bit 0 to 3 specify the color that can be scanned as the third color.  0 indicates that all colors can be scanned as the third color.

Bit 4 to 7 specify the color that can be scanned as the fourth color.  0 indicates that all colors can be scanned as the fourth color.

| Bit0-3 | Third component color |
| --- | --- |
| Bit4-7 | Fourth component color |

# Byte 10  Output Bit Depth / Dot a Color Support

This field specifies the number of bits of a single color data.  This unit supports 16bit.

| Bit0 | 1bit a color | [0] |
| --- | --- | --- |
| Bit1 | 8bit a color | [0] |
| Bit2 | 10bit a color | [0] |
| Bit3 | 12bit a color | [0] |
| Bit4 | 14bit a color | [0] |
| Bit5 | 16bit a color | [1] |
| Bit6 | Reserved | [0] |
| Bit7 | Extend bit | [0] |

# Byte 11  Number of Setup Mode

This field specifies the number of setup modes that are supported.  It becomes effective when Setup Scan2 of the Scanning Kind support field is 1.  The Setup Scan of (the number specified here + 1) types can be set.

0 is set in this unit.

# Byte 12  Digital Image Control Support

This field specifies the digital image control function that is supported.

This unit does not support the digital image control.

Byte 13  Additional length for Digital Control Information

This field specifies the additional information length (in bytes) of digital image control.

This unit sets this field to 0.

# Byte 14  Analog Control Support

This field specifies the analog image control function that is supported.

This unit supports the exposure value.

| Bit0 | Analog Gamma | [0] |
| --- | --- | --- |
| Bit1 | Exposure Time | [0] |
| Bit2 | Analog Gain | [0] |
| Bit3 | Digital Gain | [0] |
| Bit4 | Analog Shift | [0] |
| Bit5 | Analog Offset | [0] |
| Bit6 | Exposure Value | [1] |
| Bit7 | Extend bit | [0] |

# Byte 15  Additional length for Analog Control Information

This field specifies the analog image control function that is supported.

This unit sets this field to 9.

# Byte 16 to 24  First support function control parameter

The first support function in this unit is the exposure value control.

Byte 16  Bytes a Value for the Control

This specifies the length (in bytes) of the parameter for this function.

The unit of the integral time setting is 10 nsec.

# Byte 17 to 20  Minimum Value for the First Control

This specifies the minimum value that can be set.

The minimum value of the integral time in this unit is 10 nsec.

# Byte 21 to 24  Maximum Value for the First Control

This specifies the maximum value that can be set.

The maximum value of the integral time in this unit is 03FFFFFFh*10 nsec.

# Byte 25  Filter Support

This unit does not support the filter.

# Byte 26  Matrix Support

This unit does not support the matrix.

# Byte 27  Halftone Support

This field specifies the halftone that is supported.

This unit does not support the halftone.

2-2-2-5. Other information page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [E1h] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [35d=23h] |  |  |  |  |  |  |  |
| 4, 5 | Host cooperation function (initiator cooperation execution processing) Byte 4 [83h] Byte 5 [05h] |  |  |  |  |  |  |  |
| 6 to 10 | Send/Read supported information (SEND/READ command support data transfer) Byte 6 [ACh] Byte 7 [00h] Byte 8 [D0h] Byte 9 [3Ah] Byte 10 [48h] |  |  |  |  |  |  |  |
| 11 | Bits per a halftone mask parameter [0] |  |  |  |  |  |  |  |
| 12 | X bit depth of Download LUT (The number of bits in the input data of the LUT that is downloaded from the initiator) [0] |  |  |  |  |  |  |  |
| 13 | Y bit depth of Download LUT (The number of bits in the output data of the LUT that is downloaded from the initiator) [0] |  |  |  |  |  |  |  |
| 14 | Bits per a Histogram Data [0] |  |  |  |  |  |  |  |
| 15 | Bits per a Max Value Data (the number of bits of the AE maximum value) [0] |  |  |  |  |  |  |  |
| 16 | Bits per a Matrix Data [0] |  |  |  |  |  |  |  |
| 17 | Bits per a Filter Data [0] |  |  |  |  |  |  |  |
| 18 | Bits per a Shading Data (The number of bits in each data of the shading correction coefficient) [16=10h] |  |  |  |  |  |  |  |
| 19 | Bits per a Dark Current Data (The number of bits in each data of the dark voltage correction coefficient) [16=10h] |  |  |  |  |  |  |  |
| 20, 21 | Execute operation support 80 (Function that is supported by operation code 8xh of Execute) Byte 20 [03h] Byte 21 [0] |  |  |  |  |  |  |  |
| 22, 23 | Execute operation support 90 (Function that is supported by operation code 9xh of Execute) Byte 22 [06h] Byte 23 [0] |  |  |  |  |  |  |  |

| 24, 25 | Execute operation support A0 (Function that is supported by operation code Axh of Execute) Byte 24 [01h] Byte 25 [0] |
| --- | --- |
| 26, 27 | Execute operation support B0 (Function that is supported by operation code Bxh of Execute) Byte 26 [09h] Byte 27 [0] |
| 28, 29 | Execute operation support C0 (Function that is supported by operation code Cxh of Execute) Byte 28 [02h] Byte 29 [0] |
| 30, 31 | Execute operation support D0 (Function that is supported by operation code Dxh of Execute) Byte 30 [01h] Byte 31 [0] |
| 32, 33 | Execute operation support E0 (Function that is supported by operation code Exh of Execute) Byte 32 [0] Byte 33 [0] |
| 34, 35 | Execute operation support F0 (Function that is supported by operation code Fxh of Execute) Byte 34 [0] Byte 35 [0] |
| 36 | Additional Information (other additional information) [02h] |
| 37 | Volatile buffer for Initiator use (RAM buffer area) [04h] |
| 38 | Non Volatile buffer for Initiator use (non-volatile memory buffer area) [0] |

*1 When an invalid logical unit selection is performed

# Byte 4 and 5  Host cooperation function

This field specifies the processing that is executed in cooperation with the initiator.

The initiator performs the processing of the bit that is set to 1.

| Byte | Bit | Descriptions | Support |
| --- | --- | --- | --- |
| 4 | 0 | Thumbnail created by driver | 1 |
|  | 1 | Averaging multiple reading by driver | 1 |
|  | 2 | Registration gap resolved by driver | 0 |
|  | 3 | Dark voltage data created by driver | 0 |
|  | 4 | Shading calibration data created by driver | 0 |
|  | 5 | Auto Focus by driver | 0 |
|  | 6 | Shading correction by driver | 0 |
|  | 7 | Extend bit | 1 |
| 5 | 0 | Multi line simultaneous reading process by driver | 1 |
|  | 1 | Pitch in the main-scanning direction by driver | 0 |
|  | 2 | Truncated by driver | 1 |
|  | 3 | CCD Data Created by Driver | 1 |
|  | 4 to 6 | Reserved | 0 |
|  | 7 | Extend bit | 0 |

# Byte 6 to 10  Send/Read supported information

This field specifies the data transfer that is supported by the Send and the Read commands.

The data transfer of the bit that is set to 1 is supported.

| Byte 6 | Bit0 | Halftone mask reading supported | [0] |
| --- | --- | --- | --- |
|  | Bit1 | Halftone mask writing supported | [0] |
|  | Bit2 | Gamma function reading supported | [0] |
|  | Bit3 | Gamma function writing supported | [0] |
|  | Bit4 | Histogram Data reading supported | [0] |
|  | Bit5 | Max Value Data reading supported | [0] |
|  | Bit6 | Reserved | [0] |
|  | Bit7 | Extend bit | [1] |

| Byte 7 | Bit0 | Matrix Data reading supported | [0] |
| --- | --- | --- | --- |
|  | Bit1 | Matrix Data writing supported | [0] |
|  | Bit2 | Filter Data reading supported | [0] |
|  | Bit3 | Filter Data writing supported | [0] |
|  | Bit4 | Shading Data reading supported | [0] |
|  | Bit5 | Shading Data writing supported | [0] |
|  | Bit6 | Reserved | [0] |
|  | Bit7 | Extend bit | [1] |

| Byte 8 | Bit0 | Dark Voltage Data reading supported | [0] |
| --- | --- | --- | --- |
|  | Bit1 | Dark Voltage Data writing supported | [0] |
|  | Bit2 | Magnetic Data reading supported | [0] |
|  | Bit3 | Magnetic Data writing supported | [0] |
|  | Bit4 | Cooperation parameters reading supported | [1] |
|  | Bit5 | Boundary data reading supported | [1] |
|  | Bit6 | Boundary data writing supported | [1] |
|  | Bit7 | Extend bit | [1] |

| Byte 9 | Bit0 | Analog Gamma reading supported | [0] |
| --- | --- | --- | --- |
|  | Bit1 | Analog Gain reading supported | [1] |
|  | Bit2 | Digital Gain reading supported | [0] |
|  | Bit3 | Exposure Value reading supported | [1] |
|  | Bit4 | Setup Information reading supported | [1] |
|  | Bit5 | Setup Information writing supported | [1] |
|  | Bit6 | Perforation Information reading supported | [0] |
|  | Bit7 | Extend bit | [1] |

| Byte 10 | Bit0 | Boundary Type2 data reading supported | [0] |
| --- | --- | --- | --- |
|  | Bit1 | Boundary Type2 data writing supported | [0] |
|  | Bit2 | Initial WB Exposure Value reading supported | [0] |
|  | Bit3 | CCD data reading supported | [1] |
|  | Bit4 | Driver Soft Version reading supported | [0] |
|  | Bit5 | Driver Soft Version writing supported | [0] |
|  | Bit6 | Leak data reading supported | [1] |
|  | Bit7 | Extend bit | [0] |

Byte 11  Bits per a halftone mask parameter

This field specifies the length in bits of the halftone mask.  This unit sets this field to 0.

Byte 12 and 13  X/Y bit depth of Download LUT

This field specifies the length in bits of the input/output data in the LUT that is transferred from the initiator.

This unit sets this field to 0.

Byte 14  Bits per a Histogram Data

This field specifies the length in bits of each histogram data.  This unit sets this field to 0.

Byte 15  Bits per a Max Value Data

This field specifies the length in bits of the AE maximum value.  This unit sets this field to 16d.

Byte 16  Bits per a Matrix Data

This field specifies the length in bits of each matrix data.  This unit sets this field to 0.

Byte 17  Bits per a Filter Data

This field specifies the length in bits of each filter data.  This unit sets this field to 0.

Byte 18 and 19  Bits per a Shading/Dark Current Data

This field specifies the length in bits of each data for the shading correction coefficient/dark voltage correction coefficient.

This unit sets this field to 16d.

Byte 20 and 21  Execute operation support 80

This field specifies the function that is supported by operation code 8xh of EXECUTE command.

This unit supports the initialization and the return to the origin.

‘Initialize’ performs the unit initialization in the same manner as that is performed at the start of power supply.

‘Return to origin’ moves the object to be scanned or the stage (mechanical block) to the origin position.

| Byte | Bit | Operation | Value on this unit |
| --- | --- | --- | --- |
| 20 | 0 | Initialize | [1] |
|  | 1 | Return to origin | [1] |
|  | 2 to 7 | Reserved | [0] |
| 21 | 0 to 7 | Reserved | [0] |

# Byte 22 and 23  Execute operation support 90

This field specifies the function that is supported by operation code 9xh of EXECUTE command.

This unit supports the automatic execution of auto focus and the automatic execution of the shading white balance measurement.

| Byte |  | Bit |  | Operation |  | Value on this unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 |  | 0 |  | Change Unit |  | [0] |  |
|  |  | 1 |  | AF auto execute |  | [1] |  |
|  |  | 2 |  | Calibration auto execute |  | [1] |  |
|  |  | 3 to 7 |  | Reserved |  | [0] |  |
| 23 |  | 0 to 7 |  | Reserved |  | [0] |  |

# Byte 24 and 25  Execute operation support A0

This field specifies the function that is supported by operation code Axh of EXECUTE command.

This unit supports the auto focus.

| Byte | Bit | Operation | Value on this unit |
| --- | --- | --- | --- |
| 24 | 0 | Auto Focus | [1] |
|  | 1 | Color oriented Auto Focus | [0] |
|  | 2 to 7 | Reserved | [0] |
| 25 | 0 to 7 | Reserved | [0] |

# Byte 26 and 27  Execute operation support B0

This field specifies the function that is supported by operation code Bxh of EXECUTE command.

This unit supports the shading measurement and the recording of the unit-specific data setting.

| Byte |  |  | Bit |  |  | Operation |  |  | Value on this unit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 |  |  | 0 |  |  | Setup Shading Data |  |  | [1] |  |  |
|  |  |  | 1 |  |  | Setup Dark Current Correction Data |  |  | [0] |  |  |
|  |  |  | 2 |  |  | Setup Offset Correction Data |  |  | [0] |  |  |
|  |  |  | 3 |  |  | Write Data On Device Dependence |  |  | [1] |  |  |
|  |  |  | 4 |  |  | Change of Auto Unload time |  |  | [0] |  |  |
|  |  |  | 5 to 7 |  |  | Reserved |  |  | [0] |  |  |
| 27 |  |  | 0 to 7 |  |  | Reserved |  |  | [0] |  |  |

# Byte 28 and 29  Execute operation support C0

This field specifies the function that is supported by operation code Cxh of EXECUTE command.

This unit supports the focus movement.

| Byte | Bit | Operation | Value on this unit |
| --- | --- | --- | --- |
| 28 | 0 | Stage Move | [0] |
|  | 1 | Focus Move | [1] |
|  | 2 to 7 | Reserved | [0] |
| 29 | 0 to 7 | Reserved | [0] |

# Byte 30 and 31  Execute operation support D0

This field specifies the function that is supported by operation code Dxh of EXECUTE command.

This unit supports the loading/unloading of the object to be scanned.  For the movement, the absolute address specification is supported.

| Byte |  | Bit |  | Operation |  | Value on this unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30 |  | 0 |  | Unload object |  | [1] |  |
|  |  | 1 |  | Load object |  | [0] |  |
|  |  | 2 |  | Absolute positioning |  | [0] |  |
|  |  | 3 |  | Relative positioning |  | [0] |  |
|  |  | 4 |  | Rotate |  | [0] |  |
|  |  | 5 |  | Reserved |  | [0] |  |
|  |  | 6 |  | Reserved |  | [0] |  |
|  |  | 7 |  | Reserved |  | [0] |  |
| 31 |  | 0 to 7 |  | Reserved |  | [0] |  |

# Byte 32 and 33  Execute operation support E0

This field specifies the function that is supported by operation code Exh of EXECUTE command.

# Byte 34 and 35  Execute operation support F0

This field specifies the function that is supported by operation code Fxh of EXECUTE command.

# Byte 36  Additional Information

This field specifies the other additional information.

| Bit |  | Explanations of operation | Value on this unit |
| --- | --- | --- | --- |
| Bit0 | Hot exchangeable to unequipped unit with notice | The attached adapter can be exchanged with the power turned ON, and it is possible to inform the initiator that the adapter has been exchanged. | [0] |
| Bit1 | Scanned object exchangeable with notice | The scanned object can be exchanged, and it is possible to inform the initiator that the object has been exchanged. | [1] |
| Bit2 | Hot exchangeable to unequipped unit without notice | The attached adapter can be exchanged with the power turned ON, but it is not possible to inform the initiator that the adapter has been exchanged. | [0] |
| Bit3 | Scanned object exchangeable without notice | The scanned object can be exchanged, but it is not possible to inform the initiator that the object has been exchanged. | [0] |
| Bit4 to 6 | Histogram Scanning | Scanning for creating the histogram of the image data | [0] |
| Bit7 | Extend bit | Extension bit | [0] |

# Byte 37  Volatile buffer for Initiator use

This field specifies the buffer size that can be read/written freely by the initiator.  The unit is 256 bytes.

This area is preserved on the RAM of the scanner, and the written data is maintained while the power of the scanner is ON.

This unit sets this field to 4 (1 kbyte).

# Byte 38  Non Volatile buffer for Initiator use

This field specifies the buffer size that can be read/written freely by the initiator.  The unit is one byte.

This area is preserved in the non-volatile memory of the scanner, and the written data is maintained permanently.

This unit sets this field to 0, that is, this unit does not support the non-volatile memory buffer area.

2-2-2-6. Additional address information page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [C8h] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [n-3] |  |  |  |  |  |  |  |
| 4 | Number of Images |  |  |  |  |  |  |  |
| 5 to 8 | (1) Left Address |  |  |  |  |  |  |  |
| 9 to 12 | (1) Top Address |  |  |  |  |  |  |  |
| 13 to 16 | (1) Width |  |  |  |  |  |  |  |
| 17 to 20 | (1) Length |  |  |  |  |  |  |  |
| : | : |  |  |  |  |  |  |  |
| n-15 to n-12 | ((n-4)/16) Left Address |  |  |  |  |  |  |  |
| n-11 to n-8 | ((n-4)/16) Top Address |  |  |  |  |  |  |  |
| n-7 to n-4 | ((n-4)/16) Width |  |  |  |  |  |  |  |
| n-3 to n | ((n-4)/16) Length |  |  |  |  |  |  |  |

*1 When an invalid logical unit selection is performed

Byte 4  Number of Images

This field specifies the number of images that are stored in the unit.

In this unit, a value from 1 to 60 is set according to the attached holder.

Byte 5 to 8  Left Address

This field specifies the X address at the left edge of the first image.

Byte 9 to 12  Left Address

This field specifies the Y address at the top edge of the first image.

Byte 13 to 16  Width

This field specifies the width of the first image in the X-axis direction.

Byte 17 to 20  Length

This field specifies the length of the first image in the Y-axis direction.

Byte 21 and after

The address information of the second and later images is stored.

The address information of 15 images can be stored in one page.  When the number of images exceeds 15, the address information of the image is stored on the page next to page C8h.

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [C9h] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [n-3] |  |  |  |  |  |  |  |
| 4 to7 | (16) Left Address |  |  |  |  |  |  |  |
| 8 to 11 | (16) Top Address |  |  |  |  |  |  |  |
| 12 to 15 | (16) Width |  |  |  |  |  |  |  |
| 16 to 19 | (16) Length |  |  |  |  |  |  |  |
| : | : |  |  |  |  |  |  |  |
| n-15 to n-12 | ((n-4)/16 + 15) Left Address |  |  |  |  |  |  |  |
| n-11 to n-8 | ((n-4)/16 + 15) Top Address |  |  |  |  |  |  |  |
| n-7 to n-4 | ((n-4)/16 + 15) Width |  |  |  |  |  |  |  |
| n-3 to n | ((n-4)/16 + 15) Length |  |  |  |  |  |  |  |

*1 When an invalid logical unit selection is performed

2-2-2-7. CCD measurement setting page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Qualifier [0] [011b](*1) |  |  | Peripheral Device Type [6=00110b] [1Fh=11111b](*1) |  |  |  |  |
| 1 | Page code [E3h] |  |  |  |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | Page length [] |  |  |  |  |  |  |  |
| 4, 5 | Color of CCD Data |  |  |  |  |  |  |  |
| 6, 7 | Resolution of CCD Data |  |  |  |  |  |  |  |
| 8 | Scanning Number of CCD Data (The number of scanning times for the CCD measurement) |  |  |  |  |  |  |  |
| 9 | Type of CCD Data (The number of types for the CCD measurement) |  |  |  |  |  |  |  |
| 10 | A number of CCD Data [n] (The number of measurement points for the CCD measurement) |  |  |  |  |  |  |  |
| 11, 12 | First value of CCD Data (Ratio of the first point for the CCD measurement) |  |  |  |  |  |  |  |
| 13, 14 | Second value of CCD Data (Ratio of the second point for the CCD measurement) |  |  |  |  |  |  |  |
| : | : |  |  |  |  |  |  |  |
| n+10, n+11 | nth value of CCD Data (Ratio of the nth point for the CCD measurement) |  |  |  |  |  |  |  |

*1 When an invalid logical unit selection is performed

Byte 4 and 5  Color of CCD Data

This field specifies the color for the CCD measurement.  The color in which 1 is set is used for the CCD measurement.  (Two or more colors may be specified simultaneously.)

Byte 4

| Bit 0 | R [0:OFF/1:ON] |
| --- | --- |
| Bit 1 | G [0:OFF/1:ON] |
| Bit 2 | B [0:OFF/1:ON] |
| Bit 3 | NG [0:OFF/1:ON] |
| Bit 4 | C [0:OFF/1:ON] |
| Bit 5 | M [0:OFF/1:ON] |
| Bit 6 | Y [0:OFF/1:ON] |
| Bit 7 | K [0:OFF/1:ON] |

Byte 5

| Bit 0 to 7 | Reserved |
| --- | --- |

Byte 6 and 7  Resolution of CCD Data

This field specifies the resolution for the CCD measurement.

Byte 8  Scanning Number of CCD Data

This field specifies the number of scanning times for the CCD measurement.

Byte 9  Type of CCD Data

This field specifies the number of types for the CCD measurement.  The CCD measurement is necessary as many times as the number of measurement colors that is specified in byte 4 above multiplied by the number of types.

Byte 10  A number of CCD Data

This field specifies the number of measurement points for the CCD measurement.

Byte 11 and after  nth value of CCD Data

This field specifies the ratio of each point for the number of measurement points set in byte 10.

2-3. MODE SELECT (6) Command

# Table 2-3-1 MODE SELECT (6) command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [15h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | PF [1] | Reserved [0] |  |  | SP [0] |
| 2, 3 | Reserved [0] |  |  |  |  |  |  |  |
| 4 | Parameter list length [0,4,12,20] |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

- The MODE SELECT (6) command provides a means for the initiator to specify the device parameter to this unit.

- The PF (Page Format) bit must be set to 1.

It means that the MODE SELECT parameters following the header and the block descriptor(s) are structured as the pages of the related parameters and are as specified in the SCSI-2 standard.

- The SP (Save Pages) bit must be set to 0.

It means that this unit does not have the page save function.  If this bit is set to 1, this unit responds with common error 1.

- The parameter list length field specifies the length in bytes of the mode parameter list that is transferred from the initiator to this unit during the DATA OUT phase.  Setting the parameter list length to 0 indicates that the data is not transferred.  This status is not regarded as an error.  The effective values of the parameter list length are 0, 4, 12, and 20.

# Table 2-3-2 Sense data that is set in each status

| Status | Sense data | Remarks |
| --- | --- | --- |
| When an initiator sends a MODE SELECT command that changes the parameter applicable to other initiators | MODE PARAMETERS CHANGED (The MODE parameter is changed by other initiator when the multi-initiator is set.) 06h-2Ah-01h-00h | Creates the UNIT ATTENTION status for all initiators other than the initiator that issued the MODE SELECT command. |
| When a parameter list length that results in truncation of any parameter for descriptor, header, or page is specified | PARAMETER LIST LENGTH ERROR (The parameter length is illegal.) 05h-1Ah-00h-00h | Terminates with the CHECK CONDITION status. |
| - When the initiator changes the field that is not changeable as reported by this unit to the value other than the current value - When the initiator sends a MODE SELECT header, block descriptor, or page header for which a non-supported value is set in the reserved field - When the initiator sends a page of the length that is different from the parameter length reported for that page by the MODE SENSE command - When the initiator sends a parameter that has a value exceeding the support range of this unit - When the initiator sets a value other than 0 in the reserved field of the mode parameter | INVALID FIELD IN PARAMETER LIST (Some illegal data exists in the parameter.) 05h-26h-00h-00h | Terminates the MODE SELECT command with the CHECK CONDITION status without changing any mode parameter. |

- Mode parameter of this unit

# Table 2-3-2 Mode parameter header

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Mode Data Length |  |  |  |  |  |  |  |
| 1 | Medium Type [0] |  |  |  |  |  |  |  |
| 2 | Device-Specific Parameter [0] |  |  |  |  |  |  |  |
| 3 | Block Descriptor Length [0, 8] |  |  |  |  |  |  |  |

- When the MODE SENSE command is used, the Mode Data Length field specifies the length in bytes of the following data that can be transferred.  Mode Data Length does not include itself.  When using the MODE SELECT command, the Mode Data Length field is set to ‘Reserved’.

- Medium Type is always set to 0.

- Device-Specific Parameter is always set to 0.

- Block Descriptor Length specifies the length in bytes of all the block descriptors.  In this unit, 0 or 8 is set.  Block Descriptor Length of 0 means that the block descriptor is not included in the mode parameter list; however, this is not regarded as an error.

- Mode parameter block descriptor

# Table 2-3-3 Mode parameter block descriptor

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Density Code [0] |  |  |  |  |  |  |  |
| 1 | (MSB) |  |  |  |  |  |  |  |
| 2 | Number of Blocks [0] |  |  |  |  |  |  |  |
| 3 | (LSB) |  |  |  |  |  |  |  |
| 4 | Reserved [0] |  |  |  |  |  |  |  |
| 5 | (MSB) |  |  |  |  |  |  |  |
| 6 | Block Length [1] |  |  |  |  |  |  |  |
| 7 | (LSB) |  |  |  |  |  |  |  |

- Density Code field is always set to 0.

- Number of Blocks field is always set to 0.

- Block Length field is always set to 1.

- Measurement Units page

This unit supports only the Measurement Units page.

# Table 2-3-4 Measurement Units page

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | PS [0] | Reserved [0] | Operation code [03h] |  |  |  |  |  |
| 1 | Parameter length [06h] |  |  |  |  |  |  |  |
| 2 | Basic measurement unit [00h] |  |  |  |  |  |  |  |
| 3 | Reserved [0] |  |  |  |  |  |  |  |
| 4 | (MSB) |  |  |  |  |  |  |  |
| 5 | Measurement unit divisor [1200/Maximum resolution] (LSB) |  |  |  |  |  |  |  |
| 6, 7 | Reserved [0] |  |  |  |  |  |  |  |

- The Measurement Units page specifies the units of measurement used for setting the window position and for positioning an object.  The measurement units are independent of the scan resolutions in the horizontal and the vertical directions.

- The Parameters Savable (PS) bit is used only with the MODE SENSE command.  This bit is reserved for the MODE SELECT command.

This bit is set to 0 in both the MODE SENSE and the MODE SELECT commands.  (It is reserved in the MODE SELECT command by the standard.)

In other words, this unit does not have the Parameters Savable function.  When this bit is 1 in the MODE SELECT command, this unit responds with common error 2.

- The parameter length field is set to 6.

- The basic measurement unit field is set to 0.  It means that this unit uses only inches as the basic measurement unit.  When this bit is 1 in the MODE SELECT command, this unit responds with common error 2.

- The measurement unit divisor specifies the value that is necessary to correspond with the basic measurement unit.  This unit uses only 1200 and the maximum resolution value (dpi) of this unit as the measurement unit divisor.  When this field is set to a value other than 1200 or the maximum resolution value (dpi) in the MODE SELECT command, this unit responds with common error 2.  The default value is 1200.

2-4. RESERVE UNIT Command

# Table 2-4-1 RESERVE UNIT command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [16h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Third party [0] | Third party device ID |  |  | Reserved [0] |
| 2 to 4 | Reserved [0] |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

The RESERVE UNIT command is used to reserve the logical units for exclusive use of the initiator.

This command requests that the entire logical unit be reserved for the exclusive use of the initiator until the reservation is superseded by another valid RESERVE UNIT command from the initiator that made the reservation, until released by a RELEASE UNIT command from the same initiator, or released by the hard reset status or the power-on cycle.  It shall be permissible for the initiator that reserved a logical unit to reserve a logical unit again.

2-5. RELEASE UNIT Command

# Table 2-5-1 RELEASE UNIT command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [17h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Third party [0] | Third party device ID |  |  | Reserved [0] |
| 2 to 4 | Reserved [0] |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

The RELEASE UNIT command is used to release the reservation of the logical units that was made by the initiator which issued the command.

# If a valid reservation exists, this unit shall release the reservation and return GOOD status.

# Only the initiator that made the reservation can release it.  A command that attempts to release the reservation that is not currently valid is not regarded as an error.  In this case, this unit shall return GOOD status without changing any other reservation.

2-6. MODE SENSE (6) Command

# Table 2-6-1 MODE SENSE command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [1Ah] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | PF [1] | DBD [0 or 1] | Reserved [0] |  |  |
| 2 | PC |  | Page code |  |  |  |  |  |
| 3 | Reserved [0] |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

# The MODE SENSE (6) command provides a means for this unit to report the parameters to the initiator.

- For the PF (Page Format) bit, refer to the subsection describing the MODE SELECT command.

- A DBD (Disable Block Descriptors) bit of zero indicates that this unit returns the block descriptors in the mode sense data.  When this bit is set to 1, this unit does not return the block descriptors in the mode sense data.

- The PC (Page Control) field defines the type of the mode page parameter value that is returned.  The PC field is defined in table 2-6-2.  When the PC field is 3h (saved value in the SCSI-2 standard), this unit sets the sense key and the additional sense code to ILLEGAL REQUEST and SAVING PARAMETERS NOT SUPPORTED, respectively and terminates the command with the CHECK CONDITION status.

# Table 2-6-2 PC field

| Code | Parameter type |
| --- | --- |
| 00b 01b 10b | Current value Variable value Default value |

# Current value

The PC field value of 00b requests this unit to return the current parameter value for the specified page code of the logical unit.  The current values returned are:

a) The parameters set in the last successful MODE SELECT command

b) The default values if a MODE SELECT command has not been executed normally since the last power-on, hard reset condition, or the BUS DEVICE RESET message

# Variable value

When the PC field is set to 01b, the variable parameter mask value for the page code specified by this unit is returned.  The information indicating which parameters are variable in the requested page is returned.  The bit corresponding to the variable parameter is set to 1.  The bit corresponding to the parameter that is not variable by the initiator is set to 0.

In this unit, of the parameters in the Measurement Units page, all bits in the measurement unit divisor field are set to 1 and all bits in the basic measurement unit field are set to 0, because the measurement unit divisor is variable but the basic measurement unit is not.

# Default value

When the PC field is set to 10b, the default value for the page code specified by this unit is returned.  This unit sets 0 for the parameters that are not supported.

- The page code field specifies which page or pages to return.  The page code usage is defined in table 2-6-3.

Table 2-6-3 Page code usage

| Page code | Descriptions |
| --- | --- |
| 03h 3Fh | Returns the Measurement Units page Returns all pages |

An initiator may request one or all of the pages supported by this unit.  If an initiator issues a MODE SENSE command with a page code value that is not implemented by this unit, this unit shall respond with common error 1.

A page code of 3Fh indicates that all pages implemented by this unit (however, this unit supports the Measurement Units page only) shall be returned to the initiator.

If a MODE SENSE command with both the PC field and the page code field set to 0 is received, this unit returns only the mode parameter header and the block descriptor.

2-7. SCAN Command

# Table 2-7-1 SCAN command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [1Bh] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2, 3 | Reserved [0] |  |  |  |  |  |  |  |
| 4 | Transfer length [0, 1, 2, 3] |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

The SCAN command requests this unit to start the scanning operation.

- This command is an operation activation command.  When receiving this command, this unit shall send the status and then start the scanning operation.

After this command is executed, the image data reading can be performed by the READ command.

The transfer length specifies the length in bytes of the window identifier list that shall be sent during the DATA OUT phase.  If the transfer length is 0, the data is not transferred.  This is not an error.

- The window identifier list consists of the window identifiers that define the window to be scanned.  This unit has three windows for each of the default color, and R, G, and B color.  This unit performs the scanning operation for the window that is specified by the window identifier.

The default color is valid when only the default color is read.

# The sense data that is set in each status is shown in table 2-7-2.

Table 2-7-2 Sense data that is set in each status

| Status | Sense data | Remarks |
| --- | --- | --- |
| When the default color is specified with other color in the window | INVALID COMBINATION OF WINDOWS SPECIFIED 05h-2Ch-02h-00h | Terminates with the CHECK CONDITION status. |
| When the overlapped setting is performed between two or more windows (a different setting is performed in the parameter common to all windows) | INVALID COMBINATION OF WINDOWS SPECIFIED 05h-2Ch-02h-00h | Terminates with the CHECK CONDITION status. |
| When Multi Line Simultaneous Reading is set | MULTI LINE SIMULTANEOUS READING PROCESS BY DRIVER (The rearrangement processing during Multi Line Simultaneous Reading is performed by the initiator.) 09h-80h-04h-01h | The initiator cooperative action parameter is read by the READ command following the SCAN command and the rearrangement processing is performed on the initiator side based on the information. The initiator issues the SCAN command again after performing the necessary operation. |
| When Multiple Reading is set | AVERAGING MULTIPLE READING BY DRIVER (The averaging processing during Multiple Reading is performed by the initiator.) 09h-80h-02h-00h | The initiator cooperative action parameter is read by the READ command following the SCAN command and the averaging processing is performed on the initiator side based on the information. |
| When Thumbnail is set | THUMBNAIL CREATED BY DRIVER (The thumbnail image of the object to be scanned is created by the initiator.) 09h-80h-01h-04h | The initiator cooperative action parameter is read by the READ command following the SCAN command and the thumbnail is created on the initiator side based on the information. The initiator issues the SCAN command again after performing the necessary operation. |
| When the pixel composition is 8 bits and an odd value is set in the reading resolution | TRUNCATED BY DRIVER (The excess data that is sent during the odd width reading in 8 bits is deleted by the initiator.) 09h-80h-06h-00h | The SCAN command is issued again. The excess data is deleted on the initiator side by the READ command that is issued following the SCAN command. |
| If Set up Scanning is set, after the operation is activated by the SCAN command, when the completion of reading and the device internal processing operation is confirmed by the TEST UNIT READY command, and the operation is not terminated normally | LOGICAL UNIT NOT READY, CAUSE NOT REPORTABLE (The internal mechanical error occurred.) 02h-04h-02h-00h | Terminates with the CHECK CONDITION status. If the operation is terminated normally, after the operation completion is confirmed, Max Value can be read by the READ command. |
| After the SCAN command is terminated with GOOD status, until the scan preparation such as the stage movement is completed (for TEST UNIT READY) | LOGICAL UNIT IS IN PROCESS OF BECOMING READY (During the execution of the operation activation command) 02h-04h-01h-00h (During loading/ejection of the object to be scanned) 02h-04h-01h-01h (During the measurement of the correction data) 02h-04h-01h-02h (During the execution of operation for loading the object to be scanned) 02h-04h-01h-03h (During the execution of automatic shading or white balance measurement) 02h-04h-01h-04h | Terminates with GOOD status after the preparation is completed even in the scanning status. |

2-8. SEND DIAGNOSTIC Command

# Table 2-8-1 SEND DIAGNOSTIC command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [1Dh] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | PF [0 or 1] | Re-served [0] | Self Test [0 or 1] | DevOfL [0] | Unit OfL [0] |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3, 4 | Parameter list length |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

The SEND DIAGNOSTIC command performs the self-test for this unit itself.

For the self-test of this unit, ‘Parameter exists’ or ‘Parameter does not exist’ can be selected.

●If the parameter does not exist

- The page format (PF) bit is set to 0, the Self Test bit is set to 1, and the parameter list length is set to 0.

- After the power ON, when this command is received without executing the operation activation command once, the command is terminated with GOOD status if the diagnostic result is normal during power ON, or the sense data is set according to each abnormal status and the command is terminated with CHECK CONDITION status if the diagnostic result is abnormal.

- If the operation activation command was executed, in order to notice the hardware error information, the sense data is set and the command is terminated with the CHECK CONDITION status if an error information exists, and the command is terminated with GOOD status if an error information does not exist.

If an error occurs in the operation activation command, when the operation is completed, the sense data is set as shown in the table below in the CHECK CONDITION status for TEST UNIT READY.  Then the concrete error information is set in the sense data by executing this command.

If the error information is set in the sense data by this command, the error information occurred in the operation activation command is cleared.

| Status | Sense data | Remarks |
| --- | --- | --- |
| An error occurred in the operation activation command. | Logical Unit Not Ready, Cause Not Reportable (The internal mechanical error occurred.) 02h-04h-02h-00h |  |

- If the parameter exists

- The page format (PF) bit is set to 1, the Self Test bit is set to 0, and the parameter list length is set to the transferred parameter length in bytes.  This unit does not support this status.

The SCSI device off-line (DevOfl) bit and the logical unit off-line (UnitOfl) bit must be set to 0 regardless of whether the parameter exists or not.

2-9. SET WINDOW Command

# Table 2-9-1 SET WINDOW command

| Bit Byte |  | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | Operation code [24h] |  |  |  |  |  |  |  |  |  |
| 1 |  | Logical unit number [0] |  |  |  | Reserved [0] |  |  |  |  |  |
| 2 to 5 |  | Reserved [0] |  |  |  |  |  |  |  |  |  |
|  |  | (MSB) |  |  |  |  |  |  |  |  |  |
| 6 to 8 |  | Transfer length [Recommended value: 66d] |  |  |  |  |  |  |  |  |  |
|  |  | (LSB) |  |  |  |  |  |  |  |  |  |
| 9 |  | Reserved [0] | Control byte [0] |  |  |  |  |  |  |  |  |

- The SET WINDOW command provides a means for the initiator to define the windows within the scanning range of the device.  The windows are defined by each color.

- The transfer length specifies the length in bytes of the data that shall be transferred during the DATA OUT phase.  A transfer length of 0 indicates that no window parameter data shall be transferred.  This status is not regarded as an error.

- When the transfer length is shorter than the full window parameter length of this unit, the lacking part of the parameter shall be unchanged.  If the multi-byte parameter is incompletely transferred, the suspended data shall be considered invalid and the original parameter shall be unchanged.

- When the transfer length is longer than the full window parameter length, this unit responds with common error 1.  If an illegal data is included, this unit responds with common error 2.

# Table 2-9-2 SET WINDOW data header

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 to 5 | Reserved |  |  |  |  |  |  |  |
| 6 | (MSB) Window descriptor length |  |  |  |  |  |  |  |
| 7 | [Recommended value: 58d] (LSB) |  |  |  |  |  |  |  |

- The window parameter data consists of a header followed by one or more window descriptors (refer to table 2-10-3).  Each window descriptor specifies the location, size, and the scanning method of the window.

# The window descriptor length specifies the length in bytes of a single window descriptor.

2-10. GET WINDOW Command

Table 2-10-1 GET WINDOW command

| Bit Byte | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [25h] |  |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  |  | Reserved [0] |  |  |  | Single [0, 1] |
| 2 to 4 | Reserved [0] |  |  |  |  |  |  |  |  |
| 5 | Window identifier [0, 1, 2, 3] |  |  |  |  |  |  |  |  |
|  | (MSB) |  |  |  |  |  |  |  |  |
| 6 to 8 | Transfer length [Recommended value: (58*the number of windows+8)d] |  |  |  |  |  |  |  |  |
|  | (LSB) |  |  |  |  |  |  |  |  |
| 9 | Reserved [0] |  |  | Control byte [0] |  |  |  |  |  |

The GET WINDOW command reports the values that are currently set in the scanner for each window parameter.

- The Single bit specifies the type of the requested window definition information.

When this bit is set to 1, the definition information about a single window specified by byte 5 is transferred to the initiator.

When this bit is set to 0, the definition information about all the windows that are defined by SET WINDOW or that are set in this unit as the default is transferred to the initiator.

- For the parameters to which 0 (default value) is set in SET WINDOW, the values that are currently set in this unit are reported.

- The window identifier specifies the window for which the definition information is requested when the Single bit is set to 1.  The window identifier is defined in each color.  When the Single bit is 0, this byte is invalid.

- For the window identifier, the default color, R, G, B, and Neutral Gray can be set to 0, 1, 2, 3, and 4, respectively.

- The GET WINDOW data consists of a header followed by one or more window descriptors.  Each window descriptor specifies the location, size, and the scanning method of the window.

# Table 2-10-2 GET WINDOW data header

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (MSB) Window data length |  |  |  |  |  |  |  |
| 1 | [Recommended value: (58*the number of windows+6)d] (LSB) |  |  |  |  |  |  |  |
| 2 to 5 | Reserved |  |  |  |  |  |  |  |
| 6 | (MSB) Window descriptor length |  |  |  |  |  |  |  |
| 7 | [Recommended value: 58d] (LSB) |  |  |  |  |  |  |  |

The window data length specifies the length in bytes of the data that is transferred following it.  The window data length does not include itself.  Even if the allocated length is not enough to return all the GET WINDOW data, the window data length is not adjusted for sending the cut data again.

The window descriptor length specifies the window descriptor length in bytes for a single window.

Table 2-10-3 Window Descriptor Byte

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Window Identifier [0, 1, 2, 3](The default is 2.) |  |  |  |  |  |  |  |
| 1 | Reserved [0] |  |  |  |  |  |  | Auto [0] |
| 2, 3 | X Resolution [666 to 4000] |  |  |  |  |  |  |  |
| 4, 5 | Y Resolution [333 to 4000] |  |  |  |  |  |  |  |
| 6 to 9 | Upper Left X Offset (The default is 0.) |  |  |  |  |  |  |  |
| 10 to 13 | Upper Left Y Offset (The default is 0.) |  |  |  |  |  |  |  |
| 14 to 17 | Window Width |  |  |  |  |  |  |  |
| 18 to 21 | Window Length |  |  |  |  |  |  |  |
| 22 | Brightness [0] |  |  |  |  |  |  |  |
| 23 | Threshold [0] |  |  |  |  |  |  |  |
| 24 | Contrast [0] |  |  |  |  |  |  |  |
| 25 | Image Composition [2 or 5] (The default is 2.) |  |  |  |  |  |  |  |
| 26 | Pixel Composition [16d] |  |  |  |  |  |  |  |
| 27, 28 | Halftone Pattern [0] |  |  |  |  |  |  |  |
| 29 | Reverse [0] | Reserved [0] |  |  |  | Padding Type [0] |  |  |
| 30, 31 | Bit Ordering [0] |  |  |  |  |  |  |  |
| 32 | Compression Type [0] |  |  |  |  |  |  |  |
| 33 | Compression Argument [0] |  |  |  |  |  |  |  |
| 34 to 39 | Reserved [0] |  |  |  |  |  |  |  |
| 40 | Multiple Reading Number [0 to 15] (The default is 0.) |  |  |  | Color Ordering [0, 1, 2, 3] (The default is R=1, G=2, B=3) |  |  |  |
| 41 | Averag-ing 1: ON 0: OFF* | Matrix [0] | Filter [0] | Reserved [0] | Setup Mode |  |  | Object 1: Posi* 0: Nega |
| 42 | Scanning Kind (The default is 1.) |  |  |  |  |  |  |  |
| 43 | Scanning Mode (The default is 2.) |  |  |  |  |  |  |  |
| 44 | Color interleaving (The default is 2.) |  |  |  |  |  |  |  |
| 45 | AE Value (The default is 255d.) |  |  |  |  |  |  |  |
| 46 to 49 | Exposure Value [0 to 3FFFFFFh] |  |  |  |  |  |  |  |

* Default value

Each field is described below.

The values are the absolute values to the standard, and not the relative values to the current settings.

The upper byte is first in multi-byte parameters.

A value of zero specifies the default value in the device.

Byte 0

The window identifier specifies the window defined by the window descriptor.

This unit defines the window in each scanning color by using the window identifier as shown in table 2-10-4.  The default is 2.

Table 2-10-4 Relation between the window identifier and the scanning color

| Window identifier | Scanning color | Support of this unit |
| --- | --- | --- |
| 0 | Default color (G) | Yes |
| 1 | R | Yes |
| 2 | G | Yes |
| 3 | B | Yes |
| 4 | Neutral gray | No |

# Byte 2 and 3

The X Resolution field specifies the resolution in the scan line direction.

In this unit, scanning can be performed only with the resolution of (1/Line Gap Count measure) of the maximum resolution.

Therefore, if a resolution other than the recommended resolution shown in the attached material is specified, the resolution is rounded to the supported resolution for processing by the following formulas.  In this case, Rounded parameter (sense key 01h, ASC = 37h, ASCQ = 00h) is set to the sense data and the command is terminated with the CHECK CONDITION status.

Scanning pitch = Maximum resolution/Specified resolution (Fractions are rounded off)

Scanning resolution = Maximum resolution/Scanning pitch

It is possible to obtain the resolution range that can be set in this unit by the Inquiry command.  For the correspondence between the specified resolution and the scanning resolution, and the recommended resolution, refer to table 2-10-5.

The default is the maximum resolution.

Table 2-10-5 Relation between the specified resolution and the scanning resolution

| Set Window specification |  |  | Scanning at the device |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| X resolution |  |  | Scanning resolution |  | Pitch |  |
| 4000 to 2001 |  |  | 4000 |  | 1 |  |
| 2000 to 1334 |  |  | 2000 |  | 2 |  |
| 1333 to 1001 |  |  | 1333 |  | 3 |  |
| 1000 to 667 |  |  | 1000 |  | 4 |  |
| 666 to 334 |  |  | 666 |  | 6 |  |
| 333 |  |  | 333 |  | 12 |  |

# Byte 4 and 5

The Y Resolution field specifies the resolution in the base line direction.

This field is not referred to because the X-axis resolution and the Y-axis resolution are the same in this unit.

This field is ignored in the SET WINDOW command.  The same value as the X-axis resolution is returned in the GET WINDOW command.

If the X-axis resolution is changed, the Y-axis resolution also changes according to it.

# Byte 6 to 9

The Upper Left X Offset field specifies the X-axis coordinate of the upper left corner of the window.  This coordinate is shown by the value of the distance (in inch) from the left end of the object multiplied by the specified unit divisor.  It is possible to obtain the range that can be set in this unit by the Inquiry command.

The default is 0.

# Byte 10 to 13

The Upper Left Y Offset field specifies the Y-axis coordinate of the upper left corner of the window.  This coordinate is shown by the value of the distance (in inch) from the top end of the object multiplied by the specified unit divisor.  It is possible to obtain the range that can be set in this unit by the Inquiry command.

The default is 0.

[Notes]

This unit obtains the number of scanning pixels from the scanning range and the resolution that are set by the SET WINDOW command by using the formulas below.

The X offset specified by the initiator is X_off, the Y offset is Y_off, the X-axis resolution is Xdpi, the window width is W, and the window length is L.  All variables are integers.

- When the unit divisor is the maximum resolution

Scanning pitch P = (Maximum resolution/Xdpi) The fractions are rounded off.

The number of X-axis pixels = W/P  The fractions are rounded off.

The number of Y-axis lines = L/P  The fractions are rounded off.

- When the unit divisor is 1200

X-axis scanning start pixel = (X_offset*Maximum resolution)/1200  The fractions are rounded off.

Y-axis scanning start line = (Y_offset*Maximum resolution)/1200  The fractions are rounded off.

Scanning pitch P = (Maximum resolution/Xdpi)  The fractions are rounded off.

The number of X-axis pixels = (W*Maximum resolution)/(1200*P) The fractions are rounded off.

The number of Y-axis lines = (L*Maximum resolution)/(1200*P) The fractions are rounded off.

# Byte 14 to 17

The Window Width field specifies the width of the window in the scan line direction.  The window width in inch multiplied by the specified unit divisor makes this value.  The default is the maximum window width divided by the specified unit divisor.  The maximum window width of this unit can be obtained by the Inquiry command.

# Byte 18 to 21

The Window Length field specifies the length of the window in the base line direction.  The window length in inch multiplied by the specified unit divisor makes this value.  The default is the maximum window length divided by the specified unit divisor.  The maximum window length of this unit can be obtained by the Inquiry command.

Byte 25

The Image Composition field specifies the type of the image.  The image composition is shown in table 2-10-6.

The default is 02h.

# Table 2-10-6 Image composition code

| Code | Descriptions | Support |
| --- | --- | --- |
| 00h 01h 02h 03h 04h 05h 06h to FFh | Bi-level black & white Dithered/halftone black & white Multi-level black & white Bi-level RGB color Dithered/halftone RGB color Multi-level RGB color Reserved | No No Yes No No Yes |

Byte 26

The Pixel Composition field specifies the number of bits used to represent the intensity of one pixel.

16d is set in this unit.

# Byte 29

The padding bit specifies how this unit shall pad the image data transferred to the initiator if it is not an integral number of bytes.

This bit is set to 0.

Byte 30 and 31

The Bit Ordering field is set to 0.

Byte 32

The Compression Type and the Compression Argument fields are both set to 0.  The compression is not performed in this unit.

# Byte 40

- Multiple Reading Number

This field specifies whether the multiple reading is performed or not and how many times one line is scanned in the multiple reading.  The number of scanning times per one line is (the value that is set in this field + 1).

If 0 is set in this field, the number of scanning times per one line becomes one and the normal scanning instead of the multiple reading is performed.

The default is 0.  The currently set value is specified for the GET WINDOW command.

- Color Ordering

This field specifies the order for reading the color specified in this window.

Setting this field to 0 specifies the default color ordering in the unit.  The default color ordering means R=1, G=2, and B=3.

If the overlapped setting of a value other than 0 is performed in this field for two or more windows that are set when the SCAN command is received, or if 0 and a value other than 0 are set in this field, the sense data is set as shown in the table below and the SCAN command is terminated with the CHECK CONDITION status.

The currently set value is specified for the GET WINDOW command.

| Status | Sense data | Remarks |
| --- | --- | --- |
| In the case that the overlapped setting of a value other than 0 is performed in this field for two or more windows that are set when the SCAN command is received, and that 0 and a value other than 0 are set in this field | INVALID COMBINATION OF WINDOWS SPECIFIED 05h-2Ch-02h-00h | The command is terminated with the CHECK CONDITION status. |

# Byte 41

- Averaging

When 1 is set in this field, the averaging in the main scanning direction is performed.  When 0 is set in this field, the averaging is not performed.

The default is 0.  This field specifies the currently set value for the GET WINDOW command.

- Setup Mode

This field is valid when Setup Scan2 of the Scanning Kind byte on the Set Window function page in Inquiry is supported.  The Setup mode can be specified with a value shown in Number of setup mode on the Set Window function page in Inquiry set as the upper limit.

- Negative/Positive

When 1 is set in this field, the medium is positive.  When 0 is set in this field, the medium is negative.

The default is 1.  This field specifies the currently set value for the GET WINDOW command.

# Byte 42

A bit whose kind of scanning is specified should be set to 1.

The scanning kinds that are supported by this unit can be obtained by the Inquiry command.

The default is 01h.  This field specifies the currently set value for the GET WINDOW command.

When the thumbnail scanning is set, this unit terminates the first SCAN command after the SET WINDOW command with the CHECK CONDITION status and sets ‘Thumbnail created by driver’ in the sense data.  The initiator can read the parameter (Cooperation parameter) for creating the thumbnail by the READ command.  This unit executes reading at the second SCAN command.

| Scanning Kind |  |  |
| --- | --- | --- |
|  | Bit0 | Image Scanning |
|  | Bit1 | Thumbnail Scanning |
|  | Bit2 | Set up Scanning |
|  | Bit3 | Set up Scanning2 |
|  | Bit4 | Reserved [0] |
|  | Bit5 | Auto Exposure Scanning |
|  | Bit6 | AE with WB Scanning |
|  | Bit7 | Reserved [0] |

# Byte 43

A bit whose mode is specified should be set to 1.

This unit supports the normal scan, multiple reading scan, and the high-speed scan.

The default is 02h.  This field specifies the currently set value for the GET WINDOW command.

| Scan Mode Support |  |  |
| --- | --- | --- |
|  | Bit0 | High Quality Scan |
|  | Bit1 | Normal Quality Scan |
|  | Bit2 | High Speed Scan |
|  | Bit3 | Reserved [0] |
|  | Bit4 | Multiple Reading Scan |
|  | Bit5 | Reserved [0] |
|  | Bit6 | Reverse direction Scanning |
|  | Bit7 | Reserved [0] |

# Byte 44

This field specifies which ordering (pixel ordering, line ordering, plane ordering, or single-color three-line simultaneous reading) shall be used for reading.  It also specifies whether the X and Y offsets include the CCD distance for the pixel ordering and the line ordering.  A bit whose ordering is specified for reading is set to 1.

This unit supports the line ordering without CCD distance and the single-color three-line simultaneous reading.  The default is 2.  This field specifies the currently set value for the GET WINDOW command.

| Bit0 | Pixel without CCD distance |
| --- | --- |
| Bit1 | Line without CCD distance |
| Bit2 | Plane |
| Bit3 | Reserved [0] |
| Bit4 | Pixel with CCD distance |
| Bit5 | Line with CCD distance |
| Bit6 | 3 line Simultaneous reading |
| Bit7 | Reserved [0] |

# Byte 45

This field specifies the adjustment target value (AE Value) when the exposure adjustment (AE) is performed.  If Auto Exposure Scanning or AE with WB Scanning is set in byte 42, adjustment is performed so that the output value becomes the specified value when AE is executed.

The default in this unit is 255d.  This field specifies the currently set value for the GET WINDOW command.

When the parameter is 0, the default value (255d) is set.  This value is also returned for the GET WINDOW command.

# Byte 46 to 49

This field sets a value corresponding to the exposure time in units of 10 nsec.

In this unit, the analog gain and the exposure time are set according to this value.

This field specifies the currently set value for the GET WINDOW command.

The range of the value that can be set is from 0 to 3FFFFFFh.  When 0 is set, the value that is decided in the unit is returned.

2-11. READ Command

# Table 2-11-1 READ command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [28h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 | Data type code |  |  |  |  |  |  |  |
| 3 | Reserved [0] |  |  |  |  |  |  |  |
| 4 | Data type qualifier (upper byte) |  |  |  |  |  |  |  |
| 5 | Data type qualifier (lower byte) |  |  |  |  |  |  |  |
| 6 | (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | (LSB) |  |  |  |  |  |  |  |
| 9 | Reserved [0] | Control bit [0] |  |  |  |  |  |  |

# The READ command requests that this unit transfer the data to the initiator.

- Data type code

This is for distinguishing the types of data that is transferred between the initiator and this unit.  The details are shown in table 2-11-2.

- Data type qualifier

This provides a means to differentiate the data transfers of the same data type code except image data.

The data type qualifier is shown in tables 2-11-3 and -4.

In the following case, however, this unit responds with common error 1 (INVALID FIELD IN CDB (5h-24h-00h-00h)).

- When the lower byte of the data type qualifier is different from the data length in bytes shown in table 2-11-2

- Transfer length

This specifies the number of blocks of the data that is transferred by this unit to the initiator during the DATA IN phase.

(1 block = 1 byte in this unit)

The recommended values are as shown below.

| Data type code | Recommended value (Refer to table 2-11-2.) |
| --- | --- |
| 00h to 7Fh | Data length in bytes * the number of valid data |
| 80h and after | Data length in bytes * the number of valid data + header length in bytes (In the case of the magnetic data, the magnetic data header is included.) |

Table 2-11-2 Data type code (common to READ/SEND)

| Code | Descriptions | Support by this system*1 | Length in bytes of each valid data | Number of valid data (Number of elements) | Header included or not |
| --- | --- | --- | --- | --- | --- |
| 00h | Image | R | 1 or 2 | Variable | Not included |
| 02h | Halftone mask | - | - | - | - |
| 03h | LUT | R/S | 2 | 16384 | Not included |
| 80h | Histogram data | - | - | - | - |
| 81h | Maximum value | R | 2 | 1 | Included |
| 82h | Matrix data | - | - | - | - |
| 83h | Filter data | - | - | - | - |
| 84h | Shading data | - | - | - | - |
| 85h | Dark voltage correction data | - | - | - | - |
| 86h | Magnetic data | - | - | - | - |
| 87h | Initiator cooperative action parameter | R | 1 | 18 | Included |
| 88h | Boundary Information | R/S | 4 | Variable | Included |
| 89h | Analog gamma | - | - | - | - |
| 8Ah | Analog gain | R | 4 | 2 | Included |
| 8Bh | Digital gain | - | - | - | - |
| 8Ch | WB exposure value | R | 4 | 1 | Included |
| 8Dh | Reserved | - | - | - | - |
| 8Eh | Perforation information | - | - | - | - |
| 8Fh | Boundary Information Type2 | - | - | - | - |
| 90h | WB exposure value at the time of shipment | - | - | - | - |
| 91h | CCD data | R | 2 | Variable | Included |
| 92h | Driver software version information | - | - | - | - |
| 93h | Leak volume | R | 2 | 3 | Included |
| 94h-DFh | Reserved | - | - | - | - |
| E0h | Initiator RAM buffer | R/S | 1, 2, or 4 | Variable (max 1 KB) | Included |
| E1h | Initiator EEPROM buffer | - | - | - | - |

*1 R means that the code is supported only for the READ command.  R/S means that the code is supported for both the READ and the SEND commands.

*2 The valid number of pixels for CCD is 10000d.

# Table 2-11-3 Data type qualifier (upper byte)

|  | Code | Descriptions |
| --- | --- | --- |
| When the data type code is 03h, 80h, 81h, 84h, 85h, or 8Ch | 00h 01h 02h 03h | Default color (G-component element) R-component element G-component element B-component element |
| Case other than the above | **h | No meaning |

# Table 2-11-4 Data type qualifier (lower byte)

| Code | Descriptions |
| --- | --- |
| 00h 01h 02h 03h 04h and after | 1-byte data 2-byte data Reserved 4-byte data Reserved |

When this unit sends data that is smaller than the transfer length, the CHECK CONDITION status is returned. Set the ILI bit to 1, valid bit to 1, and information byte to the difference between the requested transfer length and the number of blocks actually transferred.  For image data transfer, the response shall be made in the same manner.

When the data type is 00h (image), in the READ command issued next to a READ command, the data is transferred starting from the end of the image data that is transferred by the previous READ command.  When the data type code is other than 00h, the data is transferred again starting from the top of the data (the top of the header, if any).

This command terminates with the RESERVATION CONFLICT status when there is a reserved access conflict and there is no data to be transferred.

The sense data set in each status is shown in the table below.

# Table 2-11-5 Sense data set in each status

| Status | Sense data | Remarks |
| --- | --- | --- |
| - When the READ command of the image is received without receiving the SCAN command - When the READ command is received after all image data is transferred | COMMAND SEQUENCE ERROR (A command that makes the previous SCAN command invalid is received while the scanning operation is valid) 05h-2Ch-00h-00h | The command terminates with the CHECK CONDITION status. |

2-11-1. 2-byte data transfer

For the 2-byte data, upper byte and lower byte are transferred, in that order.  For the data of three bytes or more, the upper byte, the middle byte, and the lower byte are transferred, in that order.

2-11-2. Data header

If the data type code is over 80h, the following READ data header is added at the top of the valid data.

# Table 2-11-6 READ data header

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Data type code |  |  |  |  |  |  |  |
| 1 | The number of bits in each valid data |  |  |  |  |  |  |  |
|  | (MSB) |  |  |  |  |  |  |  |
| 2 to 5 | Valid data length |  |  |  |  |  |  |  |
|  | (LSB) |  |  |  |  |  |  |  |

- Data type code

This field specifies the type of data, and the same value as the data type for the READ command is set.

- The number of bits in each valid data

This field specifies the length in bits of the valid data to be transferred.

For example, in the case of the 14-bit data, the transferred data is 2 bytes (=16 bits).  However, because the valid data is 14 bits, 0Eh (=14d) is returned for this byte.

- Valid data length

This field specifies the length in bytes of the valid data that is currently retained in the unit.  The valid data length that is handled as the standard in this unit is the length in bytes of each valid data multiplied by the number of valid data in table 2-11-2.

In this unit, when the number of valid data is the fixed value, the length in bytes of each valid data multiplied by the number of valid data is set for the valid data length.  When the number of valid data is variable, the length in bytes of each valid data multiplied by the number of valid data that is currently retained is set.

Even if the data transfer length for the READ command is not enough to transfer all the valid data, the valid data length is not adjusted.

2-11-3. Image data transfer

The image data output of this unit is 16 bits.

Each data consists of 2 bytes and the data transfer is performed by setting 8 bits on the MSB side in the first byte, then the remaining 8 bits are transferred.

When the thumbnail scanning is performed for the strip film, the length is also measured at the same time.  Therefore there is no way for the initiator to check when the end of the film comes.  So it is possible that the data becomes smaller than the transfer length.  In this case, the CHECK CONDITION status shall be returned.  Set the ILI bit to 1, valid bit to 1, and information byte to the difference between the requested transfer length and the number of blocks actually transferred.  After that, the following sense data shall be returned when the next READ command is received.

| Status | Sense data | Remarks |
| --- | --- | --- |
| When the READ command is received after all the image data is transferred | COMMAND SEQUENCE ERROR (A command that makes the previous SCAN command invalid is received while the scanning operation is valid) 05h-2Ch-00h-00h | The command terminates with the CHECK CONDITION status. |

2-11-3-1. Image data format

The format of the transferred data changes variously depending on the setting for the ASIC during the scanning.

Some of the examples of the one-line data formats in the typical setting are shown below.  The data in the following format is repeated as many times as the number of scanning lines (sub-scanning).

- When the three colors are output in the order of R, G, and B in line ordering and transferred

| R1 H | R1 L | … | Rn H | Rn L | G1 H | G1 L | … | Gn H | Gn L | B1 H | B1 L | … | Bn H | Bn L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- When the three colors are output in the order of R, G, and B in line ordering and transferred for three-line reading

| Line1 R1 H | Line1 R1 L | Line2 R1 H | Line2 R1 L | Line3 R1 H | Line3 R1 L | … | Line1 Rn H | Line1 Rn L | Line2 Rn H | Line2 Rn L | Line3 Rn H | Line3 Rn L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Line1 G1 H | Line1 G1 L | Line2 G1 H | Line2 G1 L | Line3 G1 H | Line3 G1 L | … | Line1 Gn H | Line1 Gn L | Line2 Gn H | Line2 Gn L | Line3 Gn H | Line3 Gn L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Line1 B1 H | Line1 B1 L | Line2 B1 H | Line2 B1 L | Line3 B1 H | Line3 B1 L | … | Line1 Bn H | Line1 Bn L | Line2 Bn H | Line2 Bn L | Line3 Bn H | Line3 Bn L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

2-11-4. LUT

This unit does not support READ/SEND of the LUT.

2-11-5. Initiator cooperative action parameter

The READ command specifying data type code 87h is sent from the host when one of the following scanning modes is selected: thumbnail scanning, one-line multiple reading, three-line simultaneous reading, or 8-bit odd-width reading (in the main-scanning direction).  When receiving this command, this unit sends the data that conveys each information of the unit.

The contents and the format of the data are shown below.

Operation type code

| 1 | THUMBNAIL CREATED BY DRIVER | Thumbnail scanning |
| --- | --- | --- |
| 2 | AVERAGING MULTIPLE READING BY DRIVER | Line averaging for multiple reading function |
| 4 | MULTI LINE SIMULTANEOUS READING PROCESS BY DRIVER | Rearrangement for three-line simultaneous reading function |
| 7 | CCD DATA CREATED BY DRIVER | CCD data reading |

Table 2-11-5-1  Format of THUMBNAIL CREATED BY DRIVER

| Byte | Name | Descriptions | Parameter |
| --- | --- | --- | --- |
| 0 | Type Code | Operation type code | 1 |
| 1 to 4 | Sense Data | Sense data that is set by the SCAN command | 09h-80h-01h-04h |
| 5, 6 | Bytes Per Line | The number of bytes per line | Depends on the scanning condition |
| 7, 8 | Entire Lines | The number of entire lines | Number of scanning lines*Number of frames |
| 9 | Bits Per a Color of Dot | The number of bits per dot of one color | [16d] |
| 10, 11 | Lines Per an Image | The number of lines per image | The number of scanning lines |
| 12 | Reading Count Per a Line | Exposure counts per line | - |
| 13 to 17 | Reserved | Reserved | 0 |

Table 2-11-5-2  Format of AVERAGING MULTIPLE READING BY DRIVER

| Byte | Name | Descriptions | Parameter |
| --- | --- | --- | --- |
| 0 | Type Code | Operation type code | 2 |
| 1 to 4 | Sense Data | Sense data that is set by the SCAN command | 09h-80h-02h-00h |
| 5, 6 | Bytes Per Line | The number of bytes per line | - |
| 7, 8 | Entire Lines | The number of entire lines | - |
| 9 | Bits Per a Color of Dot | The number of bits per dot of one color | - |
| 10, 11 | Lines Per an Image | The number of lines per image | - |
| 12 | Reading Count Per a Line | Exposure counts per line | Depends on the scanning condition |
| 13 to 17 | Reserved | Reserved | 0 |

Table 2-11-5-3  Format of MULTI LINE SIMULTANEOUS READING PROCESS BY DRIVER

| Byte | Name | Descriptions | Parameter |
| --- | --- | --- | --- |
| 0 | Type Code | Operation type code | 4 |
| 1 to 4 | Sense Data | Sense data that is set by the SCAN command | 09h-80h-04h-01h |
| 5, 6 | Bytes Per Line | The number of bytes per line | Depends on the scanning condition |
| 7, 8 | Entire Lines | The number of entire lines | The number of scanning lines |
| 9 | Bits Per a Color of Dot | The number of bits per dot of one color | Depends on the scanning condition |
| 10, 11 | Lines Per an Image | The number of lines per image | 0 |
| 12 | Reading Count Per a Line | Exposure counts per line | 0 |
| 13, 14 | Registration gap | Line Gap Count divided by Scanning pitch | Depends on the scanning condition |
| 15 to 17 | Reserved | Reserved | 0 |

Table 2-11-5-4  Format of CCD DATA CREATED BY DRIVER

| Byte | Name | Descriptions | Parameter |
| --- | --- | --- | --- |
| 0 | Type Code | Operation type code | 7 |
| 1 to 4 | Sense Data | Sense data that is set by the SCAN command | 09h-80h-07h-00h |
| 5 | CCD Data Type of R Data | Type for CCD measurement of R color | Depends on the scanning condition |
| 6 | CCD Data Type of G Data | Type for CCD measurement of G color | Depends on the scanning condition |
| 7 | CCD Data Type of B Data | Type for CCD measurement of B color | Depends on the scanning condition |
| 8 | CCD Data Type of NG Data | Type for CCD measurement of NG color | - |
| 9 | CCD Data Type of C Data | Type for CCD measurement of C color | - |
| 10 | CCD Data Type of M Data | Type for CCD measurement of M color | - |
| 11 | CCD Data Type of Y Data | Type for CCD measurement of Y color | - |
| 12 | CCD Data Type of K Data | Type for CCD measurement of K color | - |
| 13 to 17 | Reserved | Reserved | 0 |

Byte 5 to 12  CCD Data Type of color Data

This field specifies the type that is used for the CCD measurement of each color.

2-11-6. Boundary Information

After the thumbnail scanning of the Brownie strip film, the coordinate information of each frame is set in the unit by the host.  The boundary address (Top Address) specified by the host is represented by inch*maximum resolution (the number of lines in pitch 1).

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0, 1 | (MSB) Parameter length [n-1] (LSB) |  |  |  |  |  |  |  |
| 2 | The actual number of images |  |  |  |  |  |  |  |
| 3 | Reserved [0] |  |  |  |  |  |  |  |
| 4 to 7 | Address at the upper left of the first frame in the sub-scanning direction |  |  |  |  |  |  |  |
| 8 to 11 | Address at the upper left of the first frame in the main-scanning direction |  |  |  |  |  |  |  |
| 12 to 15 | Address at the lower right of the first frame in the sub-scanning direction |  |  |  |  |  |  |  |
| 16 to 19 | Address at the lower right of the first frame in the main-scanning direction |  |  |  |  |  |  |  |
| : | : |  |  |  |  |  |  |  |
| n-15 to n-12 | Address at the upper left of the mth frame in the sub-scanning direction |  |  |  |  |  |  |  |
| n-11 to n-8 | Address at the upper left of the mth frame in the main-scanning direction |  |  |  |  |  |  |  |
| n-7 to n-4 | Address at the lower right of the mth frame in the sub-scanning direction |  |  |  |  |  |  |  |
| n-3 to n | Address at the lower right of the mth frame in the main-scanning direction |  |  |  |  |  |  |  |

2-11-7. Analog gain

The gain value (magnification) of the analog gain is specified in the floating decimal point format of ANSI.

The gain value is specified in order of the value set in the SET WINDOW.  However, since the default value (current value) is used when 0 is set, the gain value when 0 is set is not included.

The number of parameters is the maximum number of parameters for the analog gain on the Set Window function page of the Inquiry command.

| Byte | Parameter | Descriptions |
| --- | --- | --- |
| 0, 1 | Analog gain1 | The gain value when 1 is set 1.000000000 |
| 2, 3 | Analog gain2 | The gain value when 2 is set |

2-11-8. WB exposure value

The value decided by the measurement of the unit at the time of start-up specifies the color according to the upper byte of the data type qualifier, and 4-byte data is sent for each color.

2-11-10. CCD data

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0, 1 | The first point data of the first type in CCD first line |  |  |  |  |  |  |  |
| 2, 3 | The second point data of the first type in CCD first line |  |  |  |  |  |  |  |
| ： | ： |  |  |  |  |  |  |  |
| 2(m-2), 2(m-2)+1 | The (m-1)th point data of the first type in CCD first line |  |  |  |  |  |  |  |
| 2(m-1), 2(m-1)+1 | The mth point data of the first type in CCD first line |  |  |  |  |  |  |  |
| 2m, 2m+1 | The first point data of the second type in CCD first line |  |  |  |  |  |  |  |
| 2(m+2), 2(m+2)+1 | The second point data of the second type in CCD first line |  |  |  |  |  |  |  |
| ： | ： |  |  |  |  |  |  |  |
| 2(2m-1), 2(2m-1)+1 | The mth point data of the second type in CCD first line |  |  |  |  |  |  |  |
| 2(2m), 2(2m)+1 | The first point data of the third type in CCD first line |  |  |  |  |  |  |  |
| ： | ： |  |  |  |  |  |  |  |
| 2((n-1)m+(m-2)), 2((n-1)m+(m-2))+1 | The (m-1)th point data of the nth type in CCD first line |  |  |  |  |  |  |  |
| 2((n-1)m+(m-1)), 2((n-1)m+(m-1))+1 | The mth point data of the nth type in CCD first line |  |  |  |  |  |  |  |
| 2mn, 2mn+1 | The first point data of the first type in CCD second line |  |  |  |  |  |  |  |
| 2(mn+1), 2(mn+1)+1 | The second point data of the first type in CCD second line |  |  |  |  |  |  |  |
| ： | ： |  |  |  |  |  |  |  |
| 2(mn+(n-1)m+(m-2)), 2(mn+(n-1)m+(m-2))+1 | The (m-1)th point data of the nth type in CCD second line |  |  |  |  |  |  |  |
| 2(mn+(n-1)m+(m-1)), 2(mn+(n-1)m+(m-1))+1 | The mth point data of the nth type in CCD second line |  |  |  |  |  |  |  |
| 2(2mn), 2(2mn)+1 | The first point data of the first type in CCD third line |  |  |  |  |  |  |  |
| 2(2mn+1), 2(2mn+1)+1 | The second point data of the first type in CCD third line |  |  |  |  |  |  |  |
| ： | ： |  |  |  |  |  |  |  |
| 2(2mn+(n-1)m+(m-2)), 2(2mn+(n-1)m+(m-2))+1 | The (m-1)th point data of the nth type in CCD third line |  |  |  |  |  |  |  |
| 2(2mn+(n-1)m+(m-1)), 2(2mn+(n-1)m+(m-1))+1 | The mth point data of the nth type in CCD third line |  |  |  |  |  |  |  |

2-11-11. Leak volume

Leak_g, Leak_s, and Leak_k (2 bytes each) are sent to the initiator, in that order.

‘FFFFh’ is sent for all of the three kinds when they are not recorded once in the scanner.

The host should use the default value when ‘FFFFh’ is sent.

Because the value multiplied by 1,000,000 is recorded in the scanner, the value divided by 1,000,000 should be used.

2-12. SEND Command

# Table 2-12-1 SEND command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [2Ah] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 | Data type code |  |  |  |  |  |  |  |
| 3 | Reserved [0] |  |  |  |  |  |  |  |
| 4 | Data type qualifier (upper byte) |  |  |  |  |  |  |  |
| 5 | Data type qualifier (lower byte) |  |  |  |  |  |  |  |
| 6 | (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | (LSB) |  |  |  |  |  |  |  |
| 9 | Control byte [0] |  |  |  |  |  |  |  |

# The SEND command transfers the data from the initiator to this unit.

# The data type code and the data type qualifier are defined in the READ command.

The transfer length specifies the number of blocks that are transferred from the initiator to this unit during the DATA OUT phase.  The block size is the current block size specified by the mode parameter block descriptor.  The transfer length of zero is not regarded as an error, but indicates that there is no data to be transferred.

This command shall be terminated with the RESERVATION CONFLICT status if any reserved access conflict exists and there is no data to be transferred.

2-13. ABORT Command

# Table 2-13-1 ABORT command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [C0h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 to 4 | Reserved [0] |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

The ABORT command aborts the scanning operation that is started by the SCAN command.

This command is an operation activation command.

After returning GOOD status, the abort operation is performed.

When this command is received, the scan block movement for reading the next line is not performed and the scan block stops at that position.

In order to read the image data after the above situation, the SCAN command must be executed and the image must be read again from the beginning.

Even if this command is accepted while the scanning operation is not performed, GOOD status shall be returned.

2-14. EXECUTE Command

# Table 2-14-1 EXECUTE command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [C1h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 to 4 | Reserved [0] |  |  |  |  |  |  |  |
| 5 | Control byte [0] |  |  |  |  |  |  |  |

# The EXECUTE command performs the operation specified by the SET PARAMETER command.  The EXECUTE command is an operation activation command.

The EXECUTE command performs the specified operation after returning GOOD status.

A command other than the basic command must not be issued from the same initiator to this unit before the operation termination is confirmed by the TEST UNIT READY command.

# Table 2-14-2 Sense data that is set in each status

| Status | Sense data | Remarks |
| --- | --- | --- |
| When the TEST UNIT READY command is received during operation | LOGICAL UNIT IS IN PROCESS OF BECOMING READY (During the execution of the operation activation command) 02h-04h-01h-00h (During loading/ejection of the object to be scanned) 02h-04h-01h-01h (During the measurement of the correction data) 02h-04h-01h-02h (During the execution of operation for loading the object to be scanned) 02h-04h-01h-03h (During the execution of automatic shading or white balance measurement) 02h-04h-01h-04h | The command terminates with the CHECK CONDITION status. |
| When the TEST UNIT READY command is received after operation is terminated normally | NO ADDITIONAL SENSE INFORMATION (No error) 00h-00h-00h-00h | The command terminates with GOOD status. |
| When a command other than the basic command is received from the same initiator before the operation termination is confirmed by the TEST UNIT READY command | COMMAND SEQUENCE ERROR (A command that makes the previous SCAN command invalid is received while the scanning operation is valid) 05h-2Ch-00h-00h | The command terminates with the CHECK CONDITION status. The measurement-related operation that is being performed is aborted. |
| When a command other than the basic command is received from the other initiator during operation | LOGICAL UNIT COMMUNICATION FAILURE (The command cannot be executed because the internal operation is being performed.) 0Bh-08h-00h-00h | The command terminates with the CHECK CONDITION status. The operation that is being performed continues without any influence. |
| When the operation is not terminated normally | LOGICAL UNIT NOT READY, CAUSE NOT REPORTABLE (The internal mechanical error occurred.) 02h-04h-02h-00h | The command terminates with the CHECK CONDITION status for the TEST UNIT READY command that is received after the operation is terminated. |
| When the EXECUTE command is received before the operation parameter is set by the SET PARAMETER command | COMMAND SEQUENCE ERROR (The EXECUTE command is received before the parameter is set by the SET PARAMETER command.) 05h-2Ch-00h-00h | The command terminates with the CHECK CONDITION status. |

2-15. SET PARAMETER Command

# Table 2-15-1 SET PARAMETER command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [E0h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 | Operation code |  |  |  |  |  |  |  |
| 3 to 5 | Reserved [0] |  |  |  |  |  |  |  |
| 6 | (MSB) |  |  |  |  |  |  |  |
| 7 | Parameter length [Recommended value: 13d] |  |  |  |  |  |  |  |
| 8 | (LSB) |  |  |  |  |  |  |  |
| 9 | Control byte [0] |  |  |  |  |  |  |  |

The SET PARAMETER command is used to set the parameters for the internal operation of the unit.

The operation code field specifies the code that indicates the internal operation to be set.  The initiator can obtain the operation codes supported by this unit and the parameter length for each operation code by the INQUIRY command.

The operation parameter of the length specified by the parameter length field is transferred.

This unit starts the specified operation when the EXECUTE command is received after the parameters are set by the SET PARAMETER command.

# Table 2-15-2 Operation parameter

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Color specification |  |  |  |  |  |  |  |
| 1 to 4 | First setting value |  |  |  |  |  |  |  |
| 5 to 8 | Second setting value |  |  |  |  |  |  |  |
| 9, 10 | Speed |  |  |  |  |  |  |  |
| 11 | Torque |  |  |  |  |  |  |  |
| 12 | Driving method |  |  |  |  |  |  |  |

The parameter of 2 bytes or more is transferred starting from the upper byte.

- The first setting value specifies the absolute address value when the specified operation code needs the address parameter.

When AF (auto focusing) is performed (code A0h, A1h), the address on the medium where AF is performed in the main-scanning direction is specified.  Zero in this field specifies the address zero.

This field specifies the direction when the specified operation code has no address parameter and can switch the direction.  A value of zero specifies the normal direction, and a value of one specifies the reverse direction.

This field specifies the ON/OFF switching when the specified operation code has no address parameter and need to switch ON/OFF.  A value of zero indicates the OFF status, and a value of one specifies the ON status.

- The second setting value field specifies the absolute address value when the specified operation code needs two address parameters.

When AF (auto focusing) is performed (code A0h, A1h), the address on the medium where AF is performed in the sub-scanning direction is specified.  Zero in this field specifies the address zero.

- The color specification field specifies which color is used for performing auto focus when the operation code is ’Color oriented Auto Focus’.

The setting method of color specification is the same as that for the SET WINDOW command.  This field has no meaning when the other operation code is specified.

- The speed field specifies the movement speed when the operation code of variable movement speed is set.

Zero in this field specifies the default speed in the unit.  This field has no meaning when the other operation code is specified.

- The torque field specifies the torque value when the specified operation code accepts various torques.

Zero in this field indicates the default in the unit.  This field has no meaning when the other operation code is specified.

- The driving method field specifies the drive actuation when the specified operation code accepts various driving methods.

Zero in this field indicates the default in the unit.  This field has no meaning when the other operation code is specified.

Table 2-15-3 List of operation codes

| Operation code | Internal operation to be set | Contents of operation | Valid parameters | Support of this unit |
| --- | --- | --- | --- | --- |
| 80h | Initialize | This unit is initialized in the same manner as that of power ON. | None | Yes |
| 81h | Return to the origin | Return to the origin | None | Yes |
| 91h | Auto AF | Automatic AF execution ON/OFF | 1st Val | Yes |
| 92h | Auto Calibration | Automatic calibration execution ON/OFF | 1st Val | Yes |
| A0h | Auto Focus | Performs the auto focus | 1st Val, 2nd Val | Yes |
| B0h | Setup Shading Data | Performs the shading measurement | None | Yes |
| C1h | Focus Move | Moves the scan block in the AF direction | 1st Val | Yes |
| D0h | Unload object | Unloads the object | None | Yes |

1st Val: 	First setting value

2nd Val: 	Second setting value

Color: 	Color specification

Speed: 	Speed specification

Torque: 	Torque

Drive: 	Driving method

# Table 2-15-4 Descriptions of each parameter for the operation codes

| Opera-tion code | Color specifi-cation (Color) | First setting value (1st Val) | Second setting value (2nd Val) | Speed specifi-cation (Speed) | Torque (Torque) | Driving method (Drive) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 80h | - | - | - | - | - | - | No parameter |
| 81h | - | - | - | - | - | - | No parameter |
| 91h | - | Automatic AF execution 0: OFF 1: ON | - | - | - | - |  |
| 92h | - | Automatic calibration execution 0: OFF 1: ON | - | - | - | - |  |
| A0h | - | Address on the medium where AF is performed in the main-scanning direction | Address on the medium where AF is performed in the sub-scanning direction | - | - | - |  |
| B0h | - | - | - | - | - | - | No parameter |
| C1h | - | Address in the AF direction | - | - | - | - |  |
| D0h | - | - | - | - | - | - | No parameter |

Note) The address is shown in units of 4000 dpi.

2-16. GET PARAMETER Command

# Table 2-16-1 GET PARAMETER command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [E1h] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 | Operation code |  |  |  |  |  |  |  |
| 3 to 5 | Reserved [0] |  |  |  |  |  |  |  |
| 6 | (MSB) |  |  |  |  |  |  |  |
| 7 | Parameter length |  |  |  |  |  |  |  |
| 8 | (LSB) |  |  |  |  |  |  |  |
| 9 | Control byte [0] |  |  |  |  |  |  |  |

The current settings for the operation specified by the operation code can be read by using the GET PARAMETER command.

For the details of the parameters to be transferred, refer to table 2-15-4.

The parameters of the items that are not set contain a value of zero.

Even if the specified operation code differs from the operation code that is set in the previous SET PARAMETER command, the parameter of the specified code is returned.

The data returned for each operation code is the value that is currently set in the unit.

2-17. RECEIVE DIAGNOSTIC RESULTS Command

Table 2-17-1 RECEIVE DIAGNOSTIC RESULTS command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation code [1Ch] |  |  |  |  |  |  |  |
| 1 | Logical unit number [0] |  |  | Reserved [0] |  |  |  |  |
| 2 | Reserved [0] |  |  |  |  |  |  |  |
| 3 | (MSB) Allocation length (LSB) |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 | Reserved [0] |  |  |  |  |  |  |  |

The RECEIVE DIAGNOSTIC RESULTS command is used to read the diagnostic data after the SEND DIAGNOSTIC command is executed.  When this command is received, this unit transfers the diagnostic results of the SEND DIAGNOSTIC command that was executed previously to the initiator.

[Caution]

It is recommended that this unit be reserved in order to guarantee that the information about the diagnostic results will not be broken by the command output from the other initiator.

2-17-1. Error handling

Table 2-17-2 Error handling

| Status | Sense data | Remarks |
| --- | --- | --- |
| When the RECEIVE DIAGNOSTIC RESULTS command is received independently | COMMAND SEQUENCE ERROR 05h-2Ch-00h-00h | The command terminates with the CHECK CONDITION status. |